Is SEA worth it?  Short-term costs v. long-term benefits of strategic environmental assessment

Riki Therivel, Levett-Therivel sustainability consultants and Oxford Brookes University

28A North Hinksey Lane, Oxford OX2 0LX, UK   riki@levett-therivel.co.uk

Ainhoa González, University College Dublin

School of Geography, University College Dublin, Ireland   ainhoa.gonzalez@ucd.ie

Abstract

This paper itemises the costs and benefits associated with SEA, explores the extent that these can be monetised, and compares the monetary benefits of SEA to its monetary costs. The costs of SEA are definite, monetisable and short-term, and accrue to planners and developers. They include costs rarely reported in the international literature (e.g. public consultation, legal review). The benefits of SEA include environmental risk avoidance, environmental improvements, smoother project approval, and greater plan transparency: they are longer-term, much harder to monetise, and accrue mostly to the public and to the environment. The main potential benefit of SEA, which can be monetised (albeit with large uncertainties) through ecosystem services approaches, is in the avoidance of environmental harm/risk. These benefits can outweigh the SEA costs by orders of magnitude: however, they are only achieved if the SEA leads to meaningful plan changes. The ratio of costs and benefits is important as countries explore ways of ‘streamlining’ their environmental assessment processes.

Highlights

The benefits of SEA can outweigh its costs by orders of magnitude.

Some SEA benefits can be quantified using ecosystem service approaches.

SEA costs are mostly short-term and benefits are mostly long-term.

Most SEA costs accrue to planners and developers; most benefits accrue to the public and environment.

SEA must start early, and focus on alternatives and mitigation.

1. Introduction

This paper attempts to examine in more detail than ever before whether the benefits of Strategic Environmental Assessment (SEA) outweigh its costs, or whether SEA practitioners and planners are spending much money and time on carrying out SEA without commensurate benefits in terms of more environmentally friendly development, a smoother planning process, greater plan transparency and the like. It also explores who pays the costs of SEA and gets its benefits. This is framed, in particular, in terms of European SEA practice as the existing regulatory framework not only defines much of the expected costs and benefits, but has also supported significant research and literature in the area.

European studies that have explored the costs of SEA are typically based on surveys of planners (e.g. Cepuš et al. 2019; COWI 2009; EPA 2019; EU 2016, 2019; González et al. 2019; João and McLauchlan 2014; Partidario and Monteiro 2019; Therivel 2019).  The studies have focused on the cost of SEA consultants and/or the time spent by planners in preparing SEA reports.  Some have also considered the costs of public participation in SEA (e.g. Gauthier et al. 2011), and of plan delay resulting from SEA (Kontić and Dermol 2015).  The costs of SEA can generally be monetised.

However, studies of SEA benefits are much more limited. The international literature suggests that, in theory, SEA can speed up plan-making, reduce the environmental impacts and improve the environmental benefits of plans, act as an instrument of knowledge brokerage, and improve public participation in planning (e.g. Morrison-Saunders et al. 2015; Therivel 2010; van Doren et al. 2013).  However, only a very few studies explore, much less quantify, such real-life benefits of SEA.

Kontić and Dermol (2015) suggested that the benefits of reduced congestion and traffic accidents due to a new bypass would be in the range of €155,000 – €2 million per year; and they identified the costs of habitat creation/restoration (related to Habitat Directive assessment) as being €52 – 77 million.  They concluded that the cost of the SEA significantly exceeded its benefits.  However, they made no attempt to monetise the benefits of the internationally important habitat that was being protected; used environmental quality as the only dimension of benefits; and seemed to blame SEA for several years of arguably political delays in choosing a project route.  Rega et al. (2018) gave the example of the development plan for Bra (Piedmont), where the SEA authority requests included the removal of four residential areas that had been planned for an area with fertile agricultural soil; increased green buffer zones near major roads; and offset measures for woodland clearance.  However, they did not quantify the costs and benefits of these changes.

These analyses all stopped comparatively early in the ‘life’ of an SEA.  Kontić and Dermol’s (2015) analysis stopped before the planning process had concluded.  Rega et al. (2018) did not specify whether the SEA authorities’ recommendations were implemented in the plans.  Surveys of planners, such as that of João and McLauchlan (2014) which asked planners whether they would carry out SEA if it was not compulsory, are unlikely to take into account the full range of later benefits.  However, the end-date of the analysis will affect any discussion of SEA costs and benefits, since most SEA costs accrue in the short term – in the years before and directly after the plan is approved - whilst most benefits will accrue in the long term - in the decades (or even centuries) after plan adoption. Figure 1 presents some of these costs and benefits which are discussed in more detail in subsequent sections.  Analysing SEA costs and benefits without considering the post-adoption decades has the potential to significantly undervalue the benefits of SEA.

Figure 1. Common SEA costs and benefits and their timing.

Previous studies have not clearly identified who pays and who benefits from SEA.  SEA stakeholders include planning authorities, government environmental bodies, public and private developers, environmental NGOs, the wider public and the environment.  Clearly planning authorities pay for much of SEA, but who gains?

We have also found virtually no quantified or monetised analysis of the costs of legal review/challenge of the plan on the basis of an inadequate SEA.  Other benefits of SEA - including reduced cost and time for plan development and approval; improved productivity and quality; better resource management, decision-making and performance; business growth, alliances, innovation, and leadership; and support of organisational change and learning, empowerment and shared visions (Morrison-Saunders et al. 2015) – have also not been quantified or monetized in the literature.

This paper aims to itemize the costs and benefits associated with SEA, explore the extent that these can be monetized, and compare the monetary benefits of SEA to its monetary cost.  In doing so, it takes into account the full life-span of SEAs, and is mindful of the large uncertainties inherent in such an analysis.  The paper first identifies and monetizes the range of costs of SEA, and who bears the costs.  It then does the same for benefits.  Finally, it compares the costs against the benefits.

2. Methodology

SEA aims to integrate environmental considerations into plan-making, and so to promote sustainable development and environmental protection.  This paper broadly assumes an SEA process such as that required by the European SEA Directive, although the findings are likely to be universal.  Globally SEA processes generally entail the preparation of an environmental report that identifies the likely significant impacts of the plan/programme, and proposes measures to avoid or mitigate any significant negative impacts; consultation on the report with the public and government environmental bodies; taking the environmental report and consultation findings into account in plan/programme decision-making; and monitoring of the actual impacts of the plan/programme (Therivel 2010).

This paper is based on a literature review carried out for a wider research project on SEA effectiveness funded by the Irish Environmental Protection Agency in 2018-2020.  The original research on SEA effectiveness (EPA 2020) reviewed more than 180 peer-reviewed articles, using the search engines Academic Science Complete and Science Direct, and the search words ‘strategic environmental assessment’, ‘effective’, ‘effectiveness’, ‘costs’ and ‘benefits’.  These were supplemented here with literature on specific subjects (e.g. ecosystem services) relevant to the costs and benefits of SEA. These papers were qualitatively examined for references to SEA costs and benefits, both monetary or otherwise. The combined pool of identified costs and benefits were turned into ‘units’ for comparative purposes. The basis of this ‘unit’ cost is the monetary cost of preparing a typical SEA environmental report; as noted above and further elaborated in section 3.1, this is the most commonly reported, quantified and studied SEA cost. Although it is acknowledged that such costs vary across planning hierarchies (i.e. large plans commonly have larger SEA costs), this representative ‘unit’ enables comparison of costs and benefits within a plan and within a planning hierarchy – see further details below.

Some of the benefits – monetisable and non-monetisable - of the changes to plans made as a result of SEA can be determined through an analysis of ecosystem services.  Ecosystem services are the direct and indirect contributions of ecosystems to human well-being. Ecosystem services are typically organised into:

- Provisioning services: nutritional, non-nutritional material and energetic outputs from living systems as well as abiotic outputs (e.g. food, wood, water)

- Regulation and maintenance services: mediation or moderation of the ambient environment that affects human health, safety or comfort, together with abiotic equivalents (e.g. filtration, fire/erosion/flood control)

- Cultural services: non-material outputs that affect peoples’ physical and mental states (e.g. education, aesthetics) (CICES 2018).

Figure 2 summarises how monetizable and non-monetisable benefits of ecosystem services can be identified.  The benefits can be split into use (i.e. market goods and services) and non-use values (i.e. non-market goods and services).  Different techniques can be used to calculate the value of each of these (see Figure 2).  Direct benefits/costs can usually be monetised: they are ‘tangible’.  All of the other benefits/costs are much harder to monetise: they are ‘intangible’ (European Parliament 2015; Morrison-Saunders et al. 2015).

Figure 2.  Techniques for monetising and quantifying the benefits of ecosystem services (adapted from European Parliament 2015).

Monetisation of SEA costs and benefits involves a range of uncertainties, to the point that some analysts claim that it is not possible to do so, or to compare costs and benefits (e.g. EU 2019).  These uncertainties include:

- Understanding what would have happened if the SEA had not gone ahead.  Many planners already consider the environment in plan-making, and benefits should be attributed to the SEA only if the SEA provides ‘additionality’ (João and McLaughlan 2014; Kontić and Dermol 2015).

- Distinguishing the costs/benefits of SEA versus those of plan preparation, for instance if additional studies such as flood risk analyses inform both.

- Variations in costs/benefits from plan to plan.  For a large scale, long-term plan, the SEA costs may be high (>€500,000) but represent only a small proportion of the overall plan cost.  The opposite may hold true for a plan that covers a small area and is frequently updated.

- Difficulties in quantifying and monetizing the environmental benefits of SEA.  It is possible to assign monetary values to some ecosystem services, allowing a comparison of what would otherwise be incomparable, although there are concerns about the accuracy, replicability and general philosophy of this approach (e.g. Tinch et al. 2019; Small et al. 2017).

Given these uncertainties, this paper does not attempt to provide a detailed monetisation of SEA costs and benefits.  Rather, it takes the cost of preparing a typical environmental report as a ‘unit’ of cost, and compares the costs of other stages of SEA (public participation, legal challenge, monitoring) to this ‘unit’.  It then compares the benefits of SEA to the ‘units’ of cost.  This is done for two reasons.  First, a ‘large’ plan (e.g. national scale, long term, typically sectoral) will usually have a similarly ‘large’ SEA, with large costs and potentially large benefits.  A ‘smaller’ plan (e.g. local scale, shorter term or cyclical, typically land use) is likely to have correspondingly smaller costs and smaller benefits.  Considering the ratio of costs to benefits in the form of ‘units’ helps to even out this problem.  Second, as will become clear later in the article, the benefits of SEA can exceed its costs by several orders of magnitude, although an SEA can also have no benefits at all.  The fact that benefits can be so much greater than costs, and the circumstances under which this can be achieved, are more important than the precise costs and precise benefits.

3. Costs of SEA

Carrying out an SEA incurs short-term labour and resource costs to planning authorities from preparing the environmental report (or commissioning its preparation); carrying out any additional SEA consultation and responding to it; implementing plan changes resulting from the SEA; responding to any legal challenges triggered by the SEA; and monitoring the significant environmental effects of the plan.  The costs to government environmental bodies, the public and NGOs include reviewing the environmental report and responding to SEA consultations.  Developers – both private and public section - may face costs from implementing any avoidance and mitigation requirements resulting from SEA; and private sector developers may forego economic benefits from gaining planning permission, if SEA leads to a decision to not permit development on a site in which the developers have an interest.

3.1 Cost of preparing the environmental report

Only the cost of preparing the environmental report has been quantified at all reliably, and even then the data are limited (EU 2016). Such data are commonly derived from SEA procurement budgets, which largely reflect consultancy costs. The cited costs, in Europe, range from €2,000 to €1 million (Cepuš et al. 2019; COWI 2009; EU 2016, 2019; EPA 2019; González et al. 2019; Partidario and Monteiro 2019; Therivel 2019). The significant variation in costs reflects the type of plan, the time spent on the SEA, the day rates of consultants, the experience of all the parties concerned, and the baseline data already available (EU 2016). The cost of a typical local development (or land-use) plan SEA – which account for a large proportion of SEAs – is generally on the lower end of the above range, say roughly €5,000 to €100,000 (EU 2019).

Studies from other countries – where SEA is applied to different types of plans and SEA processes are different – suggest higher SEA costs.  For instance in Canada, where significant variations in SEA regulations exist across the provinces, three out of five SEAs covering energy plans, climate change mitigation and National Park management were completed for the ‘relatively low cost’ of less than €340,000 (Noble et al. 2019).  In Thailand, SEA costs are quoted as being between €790,000 and €1,320,000 (Chanchitpritcha et al. 2019), although this is mostly for early SEAs for sectoral national and sub-national plans: costs could be expected to reduce with increasing experience and smaller scale plans.  Pilot SEAs and SEAs required by multilateral development agencies seem to be particularly expensive because they are often carried out on large scale, contentious plans; multiple agencies and international experts are often involved; and the agencies may require significant public consultation, which adds to the cost, including travel costs. That said, only a limited number of SEAs worldwide can be expected to cost more than about €500,000 (Chanchitpricha et al. 2019; Noble et al. 2019). Alternative approaches are also being explored, particularly in emerging nations, to address such high costs. Rapid SEAs, for example, offer a cost-effective alternative amounting to less than 20% of the time and cost of conventional SEAs (Dalal-Clayton and Tarr 2015).

The literature rarely differentiates between external costs (i.e. commonly reported consultancy costs) and internal costs (i.e. staff time and other costs to the planning authority), so some SEA costs go unreported. However, research carried out for the European ‘SEA REFIT’ process (EU 2019) suggests that internal costs can amount to more than half of SEA costs.  It is also often difficult to distinguish between the work of planners in preparing a plan and in carrying out SEA, particularly where the plan is being prepared in the full knowledge of relevant environmental standards and requirements (EU 2016).

In summary, for the purposes of this article, we assume that preparation of the environmental report represents one ‘unit’ of cost.  Based on the information above, we have assumed that the ‘unit’ for a local scale, cyclical plan like a local land use plan is roughly €10,000 to €100,000.  For a national scale, one-off policy like a national energy policy, the ‘unit’ is assumed to be roughly €100,000 to €1 million.

3.2 Consultation costs

In Europe, the mandatory requirement to consult designated environmental authorities and the public incurs additional costs to the planning team over the consultation that would otherwise be required regarding the plan. In addition, the environmental authorities, individuals, non-government organisations, and businesses who want to be involved in the SEA process also face costs: of responding to SEA consultations (and in some cases paying for consultants to critically analyse the environmental report), and possibly attending workshops or inquiries. These costs can be considerable where environmental reports are long, Internet access is sporadic, group consensus on key messages needs to be sought, and/or meetings are held far away.  We did not find data specifically on this but, for instance, the 5-year Implementation Plan for an Irish national energy plan (not the SEA) cost €22,000 in legal review before it was published; this was prompted by the anticipated opposition to the plan in light of the significant public disapproval of projects that had emerged from the previous implementation plan, with some projects receiving over 48,000 submissions (M. Flynn, pers. comm. as part of EPA 2020).

In Canada, Gauthier et al. (2011) reported on the costs of involving the public in Quebecois SEAs. Agencies were given financial aid to foster public participation for five major hearings about emerging policy issues: the costs of each of the hearings, including for the SEAs, were between €500,000 and €1.4 million.  Again in Canada, where Indigenous people’s rights may be significantly affected by a plan or project, payment for environmental assessment consultation can be claimed by people who would be affected by the project, who have community or Aboriginal traditional knowledge, or who have knowledge about the project’s likely environmental effects (Government of Canada 2019).  These individuals can claim expenses up to €5,700 for reviewing and providing written comments on environmental impact statements; and up to €8,000 for attending an environmental assessment review panel (Government of Canada 2019). This gives an indication of the scale of possible costs to individual consultees.  If 30 people claimed these expenses, that would be equivalent to a large ‘unit’ of SEA costs.

In summary, on the basis of very limited data, if preparation of an environmental report costs one ‘unit’, then consultation on the environmental report can cost up to another ‘unit’.  The environmental report ‘unit’ accrues to the planning authority; the consultation ‘unit’ accrues primarily to the public and formal consultees.  While these ranges and associated uncertainties can be contested, they are illustrative of cost items which, as demonstrated in section 4, are trivial compared to possible SEA benefits.

3.3 Costs after preparation of the SEA report

SEA also imposes costs after the preparation of, and consultation on, the environmental report. These mainly relate to the need to respond to, and integrate, stakeholder feedback into the final plan. For three regional waste management plans in Ireland, for instance, integrating feedback into the SEA took 60% as long again as the stages before and during consultation (P. King, pers. comm. as part of EPA 2020).

Where individuals or NGOs seek a judicial review of a plan on the basis of an inadequate SEA, both they and the planning authority will need to pay for a lawyer and court fees. The losing side may have to pay the costs of the winning side. The other side’s costs can be considerable, and the threat of having to pay them can be a significant deterrent to challenging the adequacy of a plan and its SEA. As an indication of possible SEA-related legal costs, a UK wildlife group challenging the lack of SEA for a proposed road between Oxford and Cambridge had a €44,000 fundraising target (Virgin Money 2019).  That said, it may be possible to limit these costs. In England, for instance, it is possible to make an ‘Aarhus Convention claim’ which limits both the risk of exposure to the other side’s costs and the maximum cost that can be recovered from the other side (Six Pump Court 2015).

Where a judicial review finds that the SEA report was not legally adequate, the planning authority will incur the costs of delaying, rewriting and re-consulting on its plan (Cepuš et al. 2019; Musil and Smutný 2019). There will also be reputational and time costs. In Northern Ireland, for instance, a successful early SEA-related legal challenge (Seaport Investments Ltd. and others, 2007 NIQB 62) effectively led to a seven-year moratorium on new local plans. In the UK, a legal challenge to the SEAs for the revocation of regional spatial strategies (Cala Homes (South) Ltd v Secretary of State for Communities and Local Government [2010] EWHC 2866 (Admin)) stopped the revocation process for a year while legally-compliant SEAs were carried out.  In part because of concerns over the uncertainty and costs associated with legal challenge, many English SEAs have become very long and very detailed (Therivel 2019), although in other countries this seems to be less of a problem (e.g. Musil and Smutný 2019).

The cost of environmental monitoring for the SEA Directive has not been formally studied, in part because such monitoring often happens anyway for other reasons. One study, which acknowledges these difficulties, describes the cost of monitoring for SEA as ‘moderate’, and as a small proportion of the costs of overall environmental compliance (ICF 2016).

In summary, the SEA-related costs after the preparation of environmental reports and consultation can be broadly equivalent to another ‘unit’ of SEA costs.  These costs accrue to both the public and the planning authority.

3.4 Costs of mitigation measures

A key longer-term SEA cost is the cost to landowners/developers of more onerous environmental protection requirements.  In some cases, these costs are minimal, for instance a requirement for a slow start-up to seismic exploration for marine hydrocarbons, to give dolphins and whales a chance to move away from the survey area.  In other cases, they impose short-term costs on developers (passed on to the public) which will be recouped in the long term. For instance, a zero carbon home in the UK costs €1160 - 3200 more than a standard home but saves more than €230 per year in energy costs (Energy & Climate Intelligence Unit 2019).  In other cases, the mitigation costs can be significant, and unlikely to be directly felt as benefits to humans.  For instance, Kontić and Dermol (2015) cite the costs of habitat creation and restoration to protect endangered butterflies (under Habitats Directive requirements) as being €52 - 77 million.

A foregone benefit to private sector developers resulting from SEA may be that land is not zoned for development.  These foregone benefits can be significant. For instance, in England in 2015, land without planning permission was, on average, worth €25,000/ha whilst land with planning permission was worth €2.3 million/ha (UK Parliament 2017).  In summary, the costs of SEA mitigation measures to developers and, indirectly, the public can easily be the main costs of an SEA.  They range from zero ‘units’ to about 1,000 ‘units’.

4. Benefits of SEA

Carrying out an SEA can lead to no benefits at all, but it could have very significant benefits in terms of reduced environmental risks, enhanced environmental benefits, faster plan approval, improved plan implementation, streamlined project environmental assessment, and less tangible benefits such as greater democracy, environmental education and organisational restructuring towards more proactive environmental integration (Acharibasam and Noble 2014).  Most of these benefits accrue to the public and the environment.

4.1 Reduced environmental risk/costs

The major potential long-term benefit of SEA is in the reduction of environmental risks and associated costs.  The SEA process can lead to environmentally-harmful development sites being removed from the plan, the substitution of environmentally harmful development sites with less harmful sites, and more stringent environmental requirements for development projects. For these reasons, when done well, SEA is increasingly viewed as a mechanism to support the achievement of sustainable development goals (UNEP 2018).

Very few researchers have quantified or monetised the intangible benefits of SEA.  Kontić and Dermol (2015) have partly done so (see Section 1), but most of the literature simply concludes that SEA benefits are too difficult to quantify (e.g. EC 2019).  For this research, we identified studies that monetised, on a consistent basis, through ecosystem service analyses, the value of different habitats or different forms of environmental management.  This permits an analysis of the benefits of plan changes brought about by SEA. For instance, if SEA triggers a change from conventional to organic farming in an area, or from proposed low-density development (more land developed) to higher-density development (less land developed), then an analysis of the ecosystem services of both scenarios can give an understanding of the environmental benefits of SEA.  The analysis of changes in ecosystem services, rather than identification of specific values, has also been advocated by others (e.g. Small et al. 2017).

Table 1 shows examples of comparisons of ecosystem services.  The monetary valuation of ecosystem services is controversial and still evolving (e.g. Tinch et al. 2019).  However, few would argue that wetlands and estuaries provide many more ecosystem services than an equal area of grassland or farmland; and that ecosystems such as tropical forests and mangroves provide many more ecosystem services still.

Table 1.  Examples of monetised comparisons of ecosystem services (all in €/ha/yr).

| Habitat Type | Ecosystem services value (in €/ha/yr). |
| --- | --- |
| Two recent Chinese studies (Yuan et al. 2019; Zhou et al. 2017) use valuation evidence suggested by Xie et al. (2003) for the Tibet Plateau: |  |
| Wetland | 7,236 |
| Water | 5,147 |
| Woodland/forest | 2,446 -2,521 |
| Grassland | 811-835 |
| Farmland/cultivated land | 774-779 |
| Desert | 47-48 |
| In a US context, Mitsch et al. (2015) use the following estimates for the creation of new habitats: |  |
| Tidal marshes/mangroves | 175,221 |
| Estuaries | 26,138 |
| Inland swamps/floodplains | 23,214 |
| Lakes/rivers | 11,310 |
| Forests | 3,435 |
| Grassland | 3,766 |
| An Indonesian study (Sharma et al. 2019) found the following ecosystem values: |  |
| Intact forest | 23,376 |
| Logged/regrowth forest | 10,901 |
| Agriculture | 5,066 |
| Water bodies | 3,883 |
| Industrial plantation | 2,126 |

Despite the wide range in monetary values, it is clear that protecting sites with high ecosystem services value from development, or switching to more sustainable forms of development, avoids the loss of significant benefits. For instance, Sharma et al. (2019) suggest that a scenario of slower oil palm growth than ‘business as usual’ in West Kalimanthan could save €19 billion/yr in ecosystem services.  A New Zealand study (Sandhu et al. 2015) calculated that, if half of the arable land in Canterbury was changed from conventional to organic farming, the total monetary (e.g. food produced) and non-monetary (e.g. soil formation, pollination, aesthetics) value of the organic area would be €233 million/yr, compared to €183 million/yr for the conventional arable area.  If an SEA had stopped the UK government from scrapping its plans for zero carbon homes, new homeowners would have saved £2 billion in five years (Energy & Climate Intelligence Unit 2019). If a ‘large unit’ SEA achieved these changes, it would pay for itself in just ecosystem services in about three days (New Zealand), half an hour (UK), and less than 5 minutes (Indonesia).

Taking a smaller scale example, the SEA for the Clare County Development Plan in Ireland led to the number of coastal development sites in a plan being reduced from 11 to 2, omitting environmentally sensitive sites and thus mitigating by avoidance (B. McCarthy, pers. comm. as part of EPA 2020). Using the information from Table 1 as a basis, if the 11 initial sites were in estuaries worth €17,000/ha/yr in ecosystem services, and nine of the sites were moved to grassland worth €2,300/ha/yr in response to SEA findings (assuming that the sites would be zoned to urban development, with the loss of most of their ecosystem services), and if each development site was 10ha in size, then this would result in the avoided loss of €1,323,000/yr in ecosystem service benefits.  The SEA cost €60,000-70,000 (B. McCarthy, pers. comm.).  So under these assumptions the one SEA will have saved more than 20 times its cost every year, at no cost in terms of amount or type of development. Over a 50-year period of development, just this SEA would avoid the loss of €66 million in terms of ecosystem services, or the price of more than one thousand SEAs.

Clearly, these are highly simplified calculations and, in reality, the valuations would need to account for contextual factors and assumptions.  However, even with significantly different assumptions it would be difficult to find other similarly cost-effective legislation.  Politicians may be more amenable to the concept of ecosystem services than to ‘biodiversity’, facilitating decisions to protect the environment (OECD DAC 2010).  SEA may also be the only existing tool that can incorporate issues such as biodiversity decline and the climate emergency into plan-making.

In summary, SEA has the potential to pay for itself multiple times over in terms of ecosystem services.  In the examples above, if the SEA changes lasted 50 years with 0% discount rate (which is reasonable given that ecosystem services are being eroded, and so will become more valuable over time), the Indonesian SEA’s benefits would be more than 1,000,000 times greater than its costs; the UK SEA’s would be more than 100,000 times greater; and the New Zealand and Irish SEAs’ benefits would be 1,000-10,000 greater.  This is the key finding of this paper.

4.2 Faster plan approval, greater legal certainty

Whereas an SEA process that is not legally compliant and/or has little effect on plan-making can lead to additional costs in terms of objections and legal challenges, a legally compliant and effective SEA can help to reduce conflict and opposition to the plan and subsequent projects, and help to speed up both the plan approval and implementation processes.  It can help to identify significant negative impacts of a plan at a stage where they can still be avoided comparatively easily, rather than at a later stage of plan-making or at the EIA stage, where the costs of revising the plan are much greater (EU 2016).

SEA can also provide certainty that the plan is legally and publicly acceptable. In the UK, for instance, where planners need to demonstrate that their plan enables the delivery of sustainable development (Planning and Compulsory Purchase Act 2004, sec. 39(2)), the ‘sustainability appraisal’ (which incorporates SEA) report is typically used to do this. The SEA can ‘tell the story’ of the plan to avoid objections based on lack of plan transparency. This does not always lead to more support for the plan – indeed the SEA information can be used by objectors to critique a plan – but it can accelerate decision-making by providing information to politicians and inspectors.  SEA can lead to greater public acceptance: a good example of this comes from Austria, where participatory SEAs were carried out for successive rounds of waste management planning, with fewer public comments for each round (Arbter 2019).

4.3 Better plan implementation

Simply because SEA involves someone external to the planning team having to understand and interpret a plan for appraisal purposes, SEAs can lead to plans being rewritten to be more easily and clearly implementable. This can include the deletion of overlaps between plan policies, filling in of gaps, explanation of jargon, and simply a general ‘health check’ of the emerging plan.

In turn, a clearer plan leads to smoother implementation, with less uncertainty for (private and public sector) developers. To give an indication of what costs a clearer plan could avoid, in the London Borough of Hammersmith and Fulham, just a pre-application letter from a planner about a development of 200+ homes – a letter that could clarify the meaning of plan policies, for instance – costs €8,872, and a meeting with a planner costs €9,464 (LB Hammersmith and Fulham 2019). Conversely, if an SEA triggers plan clarifications that avoid fewer than a dozen meetings with developers, again the costs avoided of clearer plan policies can easily pay for the SEA.

4.4 Streamlined project approval

SEA can also help to speed up project approval (Annandale and Taplin 2003). SEA helps to identify and avoid projects that are environmentally unacceptable, thus avoiding the costs to developers of promoting a project that will not get planning permission, and the costs to objectors of fighting it (EU 2016). The cost of project delays can be considerable for both developers and the public.  This is illustrated by Kontić and Dermol (2015), who calculated that delays to a bypass cost between €155,000 and €1.5 million per year in pollution and traffic accidents.  Delays can also significantly affect developers’ schedules, and in extreme cases could bankrupt them.  Conversely, SEA provides assurance for developers by which to design and proceed with projects, for example in deciding on (more) sustainable locations or technologies (Arbter 2019; Cape et al. 2018; González et al. 2019).

Tiering of SEA and subsequent project EIAs is likely to be more efficient and cheaper than re-negotiating mitigation measures separately for each project (Noble et al. 2019; Tshibangu and Montaño 2019).  In Austria, for example, the SEAs for a series of Viennese waste management plans allowed for the rapid approval of a previously-contentious landfill site (Arbter 2019). Tshibangu and Montaño (2019) list several Brazilian SEAs that guided subsequent EIAs and environmental licenses, and note that this helped developers to reduce environmental risks and uncertainties. Using tiering approaches also can also allow the regulatory agency to scope out issues for subsequent site- or proposal-specific EIAs (Park 2014).  With a typical project EIA costing roughly as much as a typical SEA, an SEA that reduces the cost of ten EIAs by 10% each, or that speeds up just one project permission by a year would again pay for itself.

4.5 Other benefits

SEA has other intangible benefits such as: stimulation of environmental education and capacity building; development of predictive and evaluative models; alignment and cross-jurisdictional cooperation between government departments; establishment of environmental teams; greater accountability, legitimacy and transparency; and upholding of international environmental conventions and principles of good governance (Cape et al. 2018; EU 2019; González et al. 2019; Morrison-Saunders et al. 2015; Noble et al. 2019; SEPA 2011). SEA has also been advocated as an instrument of change towards more sustainable patterns of behaviour and development (Partidario 2015), and as a tool to support the achievement of Sustainable Development Goals (Morrison-Saunders et al. 2019; González et al., under review)

Although there can be considerable costs associated with SEA monitoring, monitoring also provides significant benefits. This include checking and verifying compliance with legislation; enabling compilation of environmental information, providing information about environmental trends, pressures and responses; and helping to identify and resolve problems of implementing environmental legislation (ICF 2016).  Monitoring can also ultimately lead to long-term savings. In Ireland, for example, an SEA recommendation was to monitor the actual effects of the construction of power transmission projects.  The collected evidence was used a-posteriori in oral hearings, incorporated into the organisation’s internal guidelines, and has improved the company’s procedures (González et al. 2019).  It is not possible to quantify, much less monetise, these benefits, but they add to the catalogue of considerable benefits of SEA.

5.  Is SEA worth it?

Recent stakeholder interviews/surveys in a range of countries concluded that the benefits of SEA slightly outweigh its costs, although not in all circumstances (Arce-Ruiz et al. 2019; Cepuš et al. 2019; Chanchitpricha et al. 2019; González et al. 2019; Partidario and Monteiro 2019; Therivel 2019; Tokarczyk-Dorociak et al. 2019).  In contrast to these generally faintly positive findings, the research carried out for this paper suggests that, for society as a whole, the benefits of SEA can vastly outweigh its costs, notably if it leads to environmental improvements in the plan.

Based on our findings of Sections 3 and 4, Table 2 summarises the possible monetary costs, costs avoided and benefits of an SEA process in figurative ‘units’ (n.b. costs avoided are also benefits of SEA).  The cost of the SEA process is definite, monetizable, and usually several times the cost of preparing the environmental report.  The costs of mitigation measures can also be significant.  Almost all of the costs are short-term.  The costs avoided plus the benefits of SEA are much harder to quantify and monetise, are long-term, and depend very much on the effectiveness of the SEA process.  SEA can have very significant benefits, many orders of magnitude greater than the SEA costs, but only if it results in changes in the plan.

Table 2.  Possible costs, costs avoided and benefits of an SEA process

|  | Costs | Costs avoided | Benefits |
| --- | --- | --- | --- |
| Plan-making process |  |  |  |
| A. Preparation of SEA report | Costs of a typical environmental report: 1 ‘unit’: e.g. €10,000 - €100,000 for a local development plan, €100,000 - €1 million for a national sectoral plan |  | Collation of data for other parallel purposes, e.g. other contemporaneous SEAs |
| B. Consultation on SEA report | Costs of planning/ attending SEA workshops, responding to SEA consultation etc. <1 ‘unit’ |  |  |
| Post plan adoption |  |  |  |
| C. Legitimacy of plan | Potential legal challenge on the basis of inadequate SEA. 1 ‘unit’ | SEA provides evidence that the plan has been prepared in a sustainable, transparent manner. | Supports/improves democratic processes, citizen engagement |
| D. Monitoring | Costs additional to already-required plan monitoring. Roughly 0.1 ‘unit’ for every 5 year monitoring period. |  | Data for future plans, projects, SEAs. Reduced cost of carrying out the next round of SEA by <1 ‘unit’ |
| E. Environmental and social impact of plan |  | Environmental/social costs avoided through selection of more sustainable alternatives and impact mitigation. Over 50 years, between 0 and >100,000 ‘units’. | Environmental/social enhancements through selection of superior alternatives and impact enhancement. Over 50 years, between 0 and >100 ‘units’. |
| F. Plan implementation | Mitigation measures implemented by public and private sector developers Between 0 and 100 ‘units’. Benefits foregone by private sector developers. Between 0 and 1000 ‘units’. | Better plan implementation due to non-environment related feedback (e.g. clearer plan wording). Potentially >1 ‘unit’. | Faster project implementation. Potentially >1 ‘unit’. |
| G. Subsequent EIAs / projects |  | Strategic approach to mitigation reduces need to negotiate mitigation project by project. Potentially 1 ‘unit’. |  |
| H. Wider knowledge and learning |  |  | Wider understanding of environmental impacts, mitigation, collaborative processes etc. |
| Total costs and benefits of one SEA and its influence over 50 years | Total costs of an SEA are 2-3 ‘units’, plus mitigation costs of 0 – 1,000 ‘units’ | Costs avoided (which are also benefits of SEA) are worth 0 - >100,000 ‘units’ | Benefits are worth 0 – >100 ‘units’ |

At the lower end of SEA costs avoided and benefits, Table 2 is consistent with previous commentators’ opinions about SEA cost-effectiveness. However, SEA has the potential to provide several orders of magnitude more benefits (including costs avoided) than costs. So what accounts for the difference between previous studies and this one?

First, most of the previous information on SEA cost-effectiveness has come from consultants and planners, who are understandably very cognisant of the short-term costs of SEA which fall directly on them, but are much less aware of the longer-term benefits which are felt by others. Indeed, SEA consultants are often unaware of what happens once the environmental report has been submitted.  The changes that planners make to a plan in response to SEA may well be opposed by developers whose development ambitions might be foiled, thus incurring more short-term costs to the planning team.

However, another major reason for the disconnect is that many SEA processes lead to few or no changes to the plan, so that all of the costs of carrying out the SEA are incurred but none of the benefits of improving the plan are reaped. This can easily be the case if the SEA starts late, does not robustly consider alternatives and mitigation measures, or is ignored by planners and politicians (Stoeghlehner 2010).  In short, SEAs that do not do much to change the plan are probably just about ‘worth it’ in terms of a ‘sense check’ of the plan, increased transparency, and minor wording changes to the plan.  SEAs that lead to more environmentally-friendly development – more energy efficient, less traffic/pollution-generating, with a smaller footprint (particularly on land with high ecosystem services value) etc. – are totally, overwhelmingly ‘worth it’.

It is not the intention of the paper to systematically identify costs and benefits for every stakeholder.  However, Figure 3, based on Table 2, summarises who bears the costs and gains the benefits of SEA, and at what SEA stage these costs and benefits take place.  The figure shows that, before plan adoption (A-E), SEA imposes only costs, whereas after plan adoption (F and G) there is the potential for significant benefits as well as (probably lower) costs or benefits foregone.  The pre-plan adoption costs are mostly borne by planning authorities and environmental bodies.  The post-plan adoption costs and foregone benefits – which are mostly short term - are borne by developers.  The potentially much greater, long-term benefits accrue to the public and the environment, and come in the form of improved or protected environmental conditions.  Applied to development plans and the plans of privatised utilities, SEA represents a shift in ‘gain’ from the private sector to the general public.

Figure 3.  Who pays, who benefits (please see Table 2 for underlying rationale).

6.  Conclusions

This paper has shown that it is possible to identify and, in some cases, monetise the benefits of SEA.  It has also shown SEA’s benefits can vastly exceed its costs: SEA can definitely be ‘worth it’.  Developers face most of the additional costs of environmental mitigation or foregone benefits of development.  The public and the environment gain most of the benefits.

The benefit:cost ratio can be further improved by reducing costs.  This could include a clearer focus on key issues (better scoping), a move from ultra-detailed EIA-type SEAs to more strategic appraisal approaches, good guidance, increased experience of SEA especially by planners, and fewer changes to planning and SEA policy (including a halt to any SEA ‘streamlining’ that could reduce the benefits of SEA).  However, the real opportunity to improve SEAs’ benefit-cost ratios is to improve the benefits that they provide.  This includes an early start to SEA, better incorporation of SEA procedures and findings in the planning process, a clearer SEA focus on alternatives and mitigation, less political influence on plan-making to allow SEA to play its evidence-based role in informing the plan, SEA ‘champions’ in high places, quality standards for SEAs or SEA consultants, adequate resourcing of the planning and SEA processes, and robust implementation of mitigation measures and monitoring (EPA 2020).

Further similar research is needed, in part to reduce the uncertainties listed at Section 2.  SEAs that have led to significant environmental improvements in plans could be analysed, to determine the costs and benefits of these changes, and the circumstances that have led to the changes.  Ecosystem service approaches could be applied to SEA case studies, to provide a clearer indication of the value of ecosystem services protected by SEAs.  In particular, studies of SEA benefits need to be in terms of decades not years, to fully account for the fact that SEA costs are short-term but SEA benefits are long-term.

Analysis of the effectiveness of SEA in terms of environmental protection or avoidance of environmental harm is lacking at all scales (Therivel and Gonzalez 2019), and the quantification and monetization of these benefits is even sparser.  Yet identifying and quantifying these benefits is important.  Several countries are ‘streamlining’ their environmental assessment systems (e.g. Australian Government 2014; White House 2020), or are considering changes to their SEA requirements (EU 2019), and this should be informed by evidence about SEA benefit as well as costs.  Understanding of what SEA stages and aspects lead to the highest SEA benefits and the best benefit-cost ratios can also improve SEA practice.

Acknowledgements: We are very grateful to Morrison et al. (2015) for inspiring our thinking about this subject; to Steve Smith and Sarah Krisht of Aecom for their comments on the ecosystem services section of this paper; to the three reviewers who gave powerful and invaluable advice about how to improve the article; and to Tadhg O’Mahony who champions SEA in Ireland.

References

Acharibasam, J.B., B.F. Noble 2014. Assessing the impact of strategic environmental assessment.  Impact Assessment and Project Appraisal 32(3), 177-187.

Annandale, D., R. Taplin 2003. Is environmental impact assessment regulation a ‘burden’ to private firms?  Environmental Impact Assessment Review 23(3), 383-397.

Arbter, K. 2019. SEA in Austria and the Participative SEA Round Table model.  Impact Assessment and Project Appraisal 37(3-4), 188-198.

Arce-Ruiz, R.M., J.A. Soria-Lara, A. Gonzalez del Campo 2019. SEA effectiveness is Spain: Insights from practice. Impact Assessment and Project Appraisal 37(3-4), 327-333.

Australian Government Department of Industry Science, Energy and Resources 2014. Streamlining offshore oil and gas environmental approvals, www.industry.gov.au/data-and-publications/streamlining-offshore-oil-and-gas-environmental-approvals.

Cape, L., F. Retief, P. Lochner, T. Fischer, A. Bond 2018. Exploring pluralism – Different stakeholder views of the expected and realised value of strategic environmental assessment (SEA). Environmental Impact Assessment Review 69, 32-41.

Cepuš, S., K. Strmšnik, M. Harmel, A. Krajnc, M. Premelč, E. Harmel, S. Weldt 2019. The effectiveness of the SEA process in Slovenia.  Impact Assessment and Project Appraisal 37(3-4), 312-326.

Chanchitpricha, C., A. Morrison-Saunders, A. Bond 2019. Investigating the effectiveness of strategic environmental assessment in Thailand.  Impact Assessment and Project Appraisal 37(3-4), 356-368.

CICES 2018. Common International Classification of Ecosystem Services (CICES) V5.1 https://cices.eu.

COWI 2009. Study concerning the report on the application and effectiveness of the EIA Directive, report to the European Commission.  https://ec.europa.eu/environment/archives/eia/pdf/eia_study_june_09.pdf

Dalal-Clayton, D.B., P. Tarr 2015. Using rapid SEA to influence national planning in Namibia. Impact Assessment and Project Appraisal 33(4) 278-283.

Energy & Climate Intelligence Unit 2019. Zero carbon homes: How owners of new homes are paying over the odds for energy. www.architectsjournal.co.uk//Uploads/h/l/g/ECIU_Zero_Carbon_Homes.pdf.

EPA (Environmental Protection Agency) 2020. Second review of effectiveness of strategic environmental assessment in Ireland, Project Ref. 2017-NC-MS-8. Dublin: EPA. www.epa.ie/pubs/advice/ea/secondreviewofseaeffectivenessinireland.html.

EU (European Union) 2016. Study concerning the preparation of the report on the application and effectiveness of the SEA Directive (Directive 2001/42/EC), https://ec.europa.eu/environment/eia/pdf/study_SEA_directive.pdf.

EU 2019. Study to support the REFIT evaluation of Directive 2001/42/EC on the assessment of the effects of certain plans and programmes on the environment (SEA Directive).  https://ec.europa.eu/environment/eia/sea-refit.htm.

European Parliament 2015. Ecosystem services: Valuing our natural capital, www.europarl.europa.eu/RegData/etudes/BRIE/2015/551321/EPRS_BRI(2015)551321_EN.pdf

Gauthier, M., L. Simard, J-P. Waaub 2011. Public participation in strategic environmental assessment (SEA): Critical review and the Quebec (Canada) approach. Environmental Impact Assessment 31, 48-60.

González, A., C. Bullock, A. Gaughran, K. Watkin-Bourne 2019. Towards a better understanding of SEA effectiveness in Ireland.  Impact Assessment and Project Appraisal 37(3-4), 233-246.

González A., P. Gazzola, V. Onyango, V (under review). The Mutualism of Strategic Environmental Assessment and Sustainable Development Goals. Environmental Impact Assessment Review.

Government of Canada 2019. Participant funding application for an environmental assessment, www.canada.ca/en/environmental-assessment-agency/services/public-participation/participant-funding-application-environmental-assessment.html.

ICF Consulting Services 2016. Study to support the review of environmental monitoring and reporting obligations, http://ec.europa.eu/environment/legal/reporting/pdf/Costs_and_benefits_discussion_document_sept.pdf.

João, E., A. McLauchlan 2014. Would you do SEA if didn’t have to?  Reflections on acceptance or rejection of the process.  Impact Assessment and Project Appraisal 32(2), 87-97.

Kontić, B., U. Dermol 2015. Confronting reality in strategic environmental assessment in Slovenia - costs and benefits. Environmental Impact Assessment Review 50, 42-52.

London Borough of Hammersmith and Fulham 2019. Planning pre-application advice, www.lbhf.gov.uk/planning/planning-applications/planning-advice/planning-pre-application-advice

Mitsch, W.J., B. Bernal, M.E. Hernandez 2015. Ecosystem services of wetlands.  International Journal of Biodiversity Science, Ecosystem Services & Management 11(1), 1-4.

Morrison-Saunders, A., A. Bond, J. Pope, F. Retief 2015. Demonstrating the benefits of impact assessment for proponents. Impact Assessment and Project Appraisal 33(2), 108-115.

Morrison-Saunders, A., L.E. Sanchez, F. Retief, J. Sinclair, M. Doelle, M. Jones, J.A. Wessels, J. Pope 2019. Gearing up impact assessment as a vehicle for achieving the UN sustainable development goals. Impact Assessment and Project Appraisal, DOI:10.1080/14615517.2019.1677089.

Musil, M., M. Smutný 2019. Effectiveness of strategic environmental assessment in the Czech Republic.  Impact Assessment and Project Appraisal 37(3-4) 199-209.

Noble, B., R. Gibson, L. White, J. Blakeley, P. Croal, K. Nwanekezie, M. Doelle 2019. Effectiveness of strategic environmental assessment in Canada under directive-based and informal practice.  Impact Assessment and Project Appraisal 37(3-4), 344-355.

OECD DAC Network on Environment and Development 2010. Strategic environmental assessment and ecosystem services.  http://content-ext.undp.org/aplaws_publications/1769813/Strategic%20Environment%20Assessment%20and%20Ecosystem%20Service%20full%20version.pdf.

Park, J.R. 2014. The role and use of programmatic environmental impact statements and environmental assessments in fulfilling NEPA and state environmental mandates.  Rockville, MD: US Nuclear Regulatory Commission.

Partidario, M.R. 2015. A strategic advocacy role in SEA for sustainability. Journal of Environmental Assessment Policy and Management 17(1).

Partidario, M.R., M.B. Monteiro 2019. Strategic environmental assessment effectiveness in Portugal.  Impact Assessment and Project Appraisal 37(3-4), 247-265.

Rega, C., Singer, J.P., Geneletti, D. 2018. Investigating the substantive effectiveness of strategic environmental assessment of urban planning: Evidence from Italy and Spain. Environmental Impact Assessment Review 73, 60-69.

Sandhu, H.S., S.D. Wratten, R. Cullen, B. Case 2019. The future of farming: The value of ecosystem services in conventional and organic arable land. An experimental approach.  Ecological Economics 64(4), 835-848.

SEPA (Scottish Environment Protection Agency) 2011. The Scottish Strategic Environmental Assessment Review. Stirling: SEPA.

Sharma, S.K., H. Baral, Y. Laumonier, B. Okarda, H. Komarudin, H. Purnomo, P. Pacheco 2019. Ecosystem services under future oil palm expansion scenarios in West Kalimantan, Indonesia. Ecosystem Services 39, https://doi.org/10.1016/j.ecoser.2019.100978.

Six Pump Court 2015. Challenging planning permissions.  www.6pumpcourt.co.uk/wp-content/uploads/2014/10/Challenging-grant-of-permission-by-JR1.pdf

Small, N., M. Munday, I. Durance 2017. The challenge of valuing ecosystem services that have no material benefits.  Global Environmental Change 44, 57-67.

Stoeglehner, G. 2010. Enhancing SEA effectiveness: Lessons learnt from Austrian experiences I spatial planning.  Impact Assessment and Project Appraisal 28(3), 217-231.

Therivel, R. 2010. Strategic Environmental Assessment in Action, 2nd ed.  Abingdon: Routledge.

Therivel, R. 2019. Effectiveness of Local Plan SA/SEAs.  Impact Assessment and Project Appraisal 37(3-4), 266-278.

Tinch, R., N. Beaumont, T. Sunderland, E. Ozdemiroglu, D. Barton, C. Bowe et al. 2019. Economic valuation of ecosystem goods and services: a review for decision makers.  Journal of Environmental Economics and Policy 8(4), 359-378.

Tokarczyk-Dorociak, K., J.K. Kazak, A. Haładyj, S. Szewrański, M. Świąder 2019. Effectiveness of Strategic Environmental Assessment in Poland.  Impact Assessment and Project Appraisal 37(3-4), 279-291.

Tshibangu, G.M., M. Montaño 2019. Outcomes and contextual aspects of strategic environmental assessment in a non-mandatory context: The case of Brazil.  Impact Assessment and Project Appraisal 37(3-4), 334-343.

UK Parliament 2017. Land Value Capture, https://publications.parliament.uk/pa/cm201719/cmselect/cmcomloc/766/76605.htm

UNEP (United Nations Environment Programme) 2018. Assessing Environmental Impacts - A Global Review of Legislation. http://sdg.iisd.org/news/unep-reviews-global-status-of-environmental-assessments/.

van Doren, D., P.P.J. Driessen, B. Schijf, H.A.C. Runhaar 2013. Evaluating the substantive effectiveness of SEA: Towards a better understanding.  Environmental Impact Assessment Review 38, 120-130.

Virgin Money Giving 2019. Ox-Cam Expressway wildlife campaign.

https://uk.virginmoneygiving.com/charity-web/charity/displayCharityCampaignPage.action?charityCampaignUrl=expressway.

White House 2020. Remarks by President Trump on proposed National Environmental Policy Act regulations, www.whitehouse.gov/briefings-statements/remarks-president-trump-proposed-national-environmental-policy-act-regulations/.

Xie, G.D, C.X. Lu, Y.F. Leng, D. Zheng, S.C. Li 2003. Ecological assets valuation of the Tibetan Plateau.  Journal of Natural Resources 18(2), 189-196.

Yuan, K. F. Li, H. Yang, Y. Wang 2019. The influence of land-use change on ecosystem service value in Shangzhou District.  International Journal of Environmental Research and Public Health 16(8), 1321.

Zhou, J., L. Sun, S.Y. Zang, K., Wang, J.Y. Zhao, Z.X. Li, X.M. Liu, X.R. Liu 2017. Effects of the land-use change on ecosystem service value.  Global Journal of Environmental Science and Management 3(2), 121-130.

The science of monetary or economic valuation is enormously complex, with the same term (e.g. ‘tangible’) meaning different things depending on the field and author.  Here they are broadly used to distinguish between monetisable costs and all other costs and benefits of SEA.

In this case, SEA costs refer to ‘the whole SEA procedure’, which presumably includes consultation on the environmental report and responding to the consultation.

This research does not address the issue of funding constraints on planners, which often mean that the cheapest rather than the best SEA consultants are commissioned.  In turn, this might limit the energy with which the consultants propose mitigation measures.