Space, time, and sustainability: the status and future of life cycle assessment frameworks for novel biorefinery systems

Vance, C. ab*, Sweeney, J. a, Murphy, F. ab

a School of Biosystems and Food Engineering, University College Dublin (UCD), Belfield, Dublin 4, Ireland

b BiOrbic Bioeconomy SFI Research Centre, University College Dublin, Belfield, Dublin, Ireland

*Corresponding author details, charlene.vance@ucd.ie.

Abstract

For stakeholders and decision makers within the bioeconomy, it is important that sustainability assessment methodologies be holistic, reliable, and accurate. Life cycle assessment (LCA) methodologies are well-known for their ability to avoid burden shifting by considering the impacts of a product, process, or system throughout the full life cycle. However, when it comes to assessing advanced multifunctional systems within the bioeconomy i.e. biorefineries, methodological challenges arise. Such issues are discussed at length in this review paper, which include the goal, scope, and allocation methods, land use considerations, handling of biogenic carbon and emissions, impacts assessed, simplification of feedstocks and processes, regionality, and future foreground and background systems. Furthermore, the review discusses challenges in capturing social and economic impacts with LCA methodologies, with social assessments lacking data and appropriate quantitative indicators and economic assessments lacking diversity in stakeholder and cost inclusivity. Finally, this review confirms the importance of temporal factors, regional differences, and integrating multidimensional approaches to sustainability analysis, highlighting developing LCA methodologies which successfully address these areas. Methodologies to address spatial considerations include exergetic LCIA, natural capital assessment, and integration of supply chain modelling, while methodologies to address dynamic variability include process modelling integration, system dynamics modelling integration, agent-based modelling integration, consequential LCA, prospective LCA, and dynamic LCIA. Finally, extended LCC, extended LCIA, and MCDA can address challenges identified in multidimensional sustainability integration.

Highlights

- Biorefineries are part of a circular bioeconomy but should be critically assessed

- LCA can be used but must overcome methodological challenges

- Main challenges involve consideration of space, time, and overall sustainability

- Spatial and temporal issues can be addressed through systems modelling integration

- Social and economic aspects can be integrated through LCSA methodologies

Keywords: Life cycle assessment; sustainable development; circular bioeconomy; biorefinery; dynamic variability, systems modelling.

Word count: 9891 (excluding title, author names and affiliations, keywords, abbreviations, table/figure captions, acknowledgements and references).

- Introduction

Sustainable development which encompasses the three pillars of environment, economy and society [1], is commonly defined as the ability of society to meet the needs of the present without compromising the needs of the future [2]. The global bioeconomy, which consists of economic activities involving bio-technology, bio-resources and bio-ecology [3], is often considered as one solution to achieving sustainable development by reducing reliance on unsustainable resources [4]. However, the sustainability of the bioeconomy is not inherent; bio-based feedstocks can be over-exploited, which can significantly impact communities and ecosystems worldwide [5]. It is therefore important that the sustainability progress of the developing bioeconomy is monitored, which is one of the key aims of the European Union’s (EU) Bioeconomy Strategy action plan [6].

An area within the bioeconomy that is gaining considerable interest is the biorefinery concept. Biorefineries use a cascading approach to produce multiple products from one or more biomass feedstocks [7]; for example, they can produce multiple high-value products such as proteins for human food and feed [8-10], biofuels [11,12], biochemicals, and bioplastics [13,14]. Furthermore, the biorefinery concept can be fully integrated with established bio-technologies such as anaerobic digestion (AD) [15], which can produce bioenergy and fertilizer [16,17].

Within Europe, there is significant potential in developing the bioeconomy, particularly when it comes to biorefineries. First, Europe is a technologically developed region with a skilled workforce and existing infrastructure well-suited for biorefineries [18]. For example, Europe is already a world leader in biogas production, with over 18,000 AD plants in operation since 2018 [19]. Second, due to high societal awareness [20], there is a high political willpower in many European countries to combat climate change through regulatory frameworks, policies, and directives, many of which offer economic incentives for the production of bio-based materials and energy [18]. Finally, the relatively high incomes of Europeans combined with high consumer interest in ‘green’ products [18,20] leads to a willingness to pay a ‘green premium’ [21]. Thus, despite the high costs associated with developing novel biorefineries in Europe when compared to other regions [18], the market for sustainable, locally produced bio-based products is strong.

Due to fears that the developing bioeconomy will compete with existing food production [22], there is increasing interest in developing second and third generation biorefineries within Europe, which utilize non-food crops such as lignocellulosic feedstocks, wastes, engineered crops, and marine-based/aquatic feedstocks [23]. Second and third generation biorefineries have been further categorized as yellow biorefineries, which utilize dry lignocellulosic feedstocks i.e. straw, green biorefineries, which utilize wet lignocellulosic feedstocks i.e. grass, blue biorefineries, which utilize marine-based feedstocks i.e. algae, and brown or grey biorefineries, which utilize waste streams such as municipal or agricultural sludge and solid waste [15,24]. Significant potential exists for the utilization of waste feedstocks and residues within Europe, where the theoretical potential of such feedstocks has been estimated at 8500 PJ/year [25], corresponding to 15% of the total primary energy consumption (55.74 EJ) of the EU in 2020 [26].

While there is clearly biomass resource potential, technological capability, and political willpower for developing sustainable biorefineries within Europe, how to measure the sustainability of biorefineries is less obvious. The processes and supply chains involved in the biorefinery approach are multisectoral, beginning from the feedstock cultivation and ending with the product’s use, reuse or disposal [27]. Thus, to understand the sustainability of a biorefinery product, it is important to account for the cumulative impacts contributed by each sector involved in the production and utilization of that product. Life Cycle Assessment (LCA) is a well-known application of Life Cycle Thinking (LCT) which is used to assess the impacts of a product, process or system over its lifetime [28]. Most commonly, LCA refers to environmental LCA (e-LCA), which considers only environmental impacts [29]. However, life cycle thinking can also be applied to assess social and economic impacts, for example through social LCA (s-LCA) or life cycle costing (LCC) frameworks [30]. These tools are key components of life cycle management (LCM), where sustainability knowledge is used for responsible decision-making [28].

To compare different LCAs, it is important to understand the methodological choices that were made during each stage. It is also important to understand how LCA methodologies are being adapted to better evaluate the sustainability of the developing bioeconomy. This paper will reflect on recent literature, which reviews or assesses the sustainability of novel biotechnologies, focusing on those using LCA methodologies. Methodological variations will be identified and discussed, along with key issues in mainstream LCA frameworks. Social and economic aspects of sustainability will also be explored, highlighting challenges in their inclusion both generally and through LCA frameworks. Based on the findings, recommendations will be made for frameworks which effectively address the challenges associated with assessing the sustainability of novel biorefinery systems, both within the European context and beyond.

2 	Methods

To understand the role of life cycle assessment within the context of the bioeconomy and specifically, in assessing AD and biorefinery technologies, a search for appropriate literature was conducted. The search was non-systematic, searching databases ScienceDirect and Web of Science with the keywords “bioeconomy”, “biorefinery” or “anaerobic digestion”, combined with “LCA”. To find literature regarding the economic and social aspects, the keyword “LCA” was replaced by “LCC” and “social LCA/s-LCA/sLCA”, respectively.

Through the literature review, approximately 50 papers were found to include case studies of the environmental life cycle assessment of bio-based products or processes using second and third generation feedstocks. Of those, 35 studies were identified to be of a good quality, which was defined by whether there was a detailed description of the model and LCIA results which could be extracted for comparison. Few of the case studies found included social or economic indicators. Thus, the review of social and economic aspects was limited to a more general overview.

It should be noted that of the literature read, several reviews were found which also discussed LCA methodology, focusing on topics such as energy generation technologies [31], emerging technologies [32], aquaculture systems [33], land use and biodiversity loss [34], marine ecological impacts [35], simplification practices [36], normalization and weighting [37], consideration of time [38-41], and integrated sustainability assessment [42]. However, only two previous reviews were found which focused on the LCA of biorefinery systems [7,27] (Table 1). Ahlgren et al. [27] discussed issues and recommendations for the LCA of biorefinery systems through a review of existing LCA standards and guidelines, while Julio et al. [7] discussed LCA from a process system engineering perspective, focusing on the integration of process modelling software and life cycle assessment methodologies. Similar to [27], this review will discuss issues in the LCA of biorefinery systems followed by recommendations for future studies. However, this paper will use recently published case studies to further highlight methodological issues and will include issues not explored in depth in their review (sections 3.5-3.8). Similar to [7], this review will include a discussion of economic and social aspects captured by LCC and s-LCA and will discuss the challenges involved with sustainability integration. This paper will also discuss process modelling integration as a possible solution for improved life cycle assessment. However, where process modelling was the main focus of [7], in our review, process modelling is only one solution and is linked to a specific methodological issue and area for improvement within LCA (section 5.2). To address the other methodological issues identified, additional solutions will be discussed.

The review is divided into three sections; first, methodological issues in developing LCA of biorefinery systems are identified and explored (sections 3.1-3.8). Comparisons are made between case studies to provide further insights on challenges in using LCA to assess such systems. In the second section (4.1-4.3), papers considering the sustainability assessment of bioeconomic projects are discussed, focusing on social and economic aspects. Key challenges affecting the adoption of social LCAs and life cycle costing models are highlighted, as well as issues in the integration of the three dimensions. Finally, the third section (5.1-5.3) explores developments in LCA methodologies which address the challenges described in the first and second sections. This includes novel e-LCA methodologies as well as methods for incorporating different dimensions of sustainability. Recommendations are made for methodological choices and frameworks which best capture the complexities of assessing sustainable biorefinery systems.

Table 1. Comparison of topics covered in previous reviews of the LCA of biorefinery systems and this paper.

| Review paper | Challenges | Methodological solutions |
| --- | --- | --- |
| This study | - Environmental LCA (goal, scope, functional unit, allocation, land use, sea surface use, biogenic carbon and timing of emissions, simplification of processes and feedstocks, regionality, future foreground and background systems) - Social LCA - LCC - Sustainability integration | - Spatial differentiation (Exergetic LCIA, supply chain modelling) - Dynamic variability (Process modelling and simulation, system dynamics modelling, agent-based modelling, consequential LCA, prospective LCA, dynamic LCIA) - Sustainability integration (Extended LCC, extended LCIA, MCDA) |
| Ahlgren et al. [27] | - Environmental LCA (goal, scope, functional unit, allocation, land use, biogenic carbon and timing of emissions) | - Consequential LCA |
| Julio et al. [7] | - Environmental LCA (goal, scope functional unit, allocation, land use, impacts assessed, simplification of processes and feedstocks) - Social LCA - LCC - Sustainability integration | - Process modelling and simulation - Consequential LCA |

- Current challenges in standardized LCA methodology

Life cycle assessments are standardized by the International Organization for Standardization [ISO] [43,44] and the International reference Life Cycle Data system [ILCD] [45]. The standard LCA consists of 4 stages: goal and scope definition, inventory analysis, impact assessment, and interpretation. In general, LCA faces obstacles such as over-simplification caused by a lack of process knowledge or available data and inconsistencies caused by varying methodologies leading to subjective choices [29]. In the case of biorefineries, challenges are seen in; 1) goal, scope and functional unit definition, 2) allocation of feedstock and biorefinery outputs, 3) land use considerations and 4) handling of biogenic carbon and timing of emissions [27]. Additional considerations which have been identified are; 5) impacts assessed, 6) feedstock and process simplification, 7) regionality and 8) effects of future foreground and background systems. These challenges can be categorized by the affected stage of the methodology, as seen in Figure 1, and will be further expanded on in the following subsections.

Figure 1. LCA stages [43] with the key methodological challenges for assessing biorefinery systems.

3.1 Goal, scope and functional unit definition

While the ISO frameworks specify that LCAs should have a clear definition of goal and scope [43,44], it is in this stage that methodological divergence begins. In the goal definition, the intended applications, limitations, reasons for study, target audience, comparative studies and stakeholders should be defined [46]. The scope definition then determines what product systems will be assessed and how the assessment will be carried out [47]. The goal and scope of an LCA will determine what type of LCA is applied. LCA can be partial (cradle-to-gate, gate-to-gate, gate-to-grave), or full (cradle-to-grave, cradle-to-cradle). An attributional LCA (A-LCA) aims to assess all stages of the life cycle, while a consequential LCA (C-LCA) aims to identify consequences of a life cycle change on other systems [48]. For biorefineries, two groups of LCA studies can be identified: those whose goals focus on comparing biomass feedstocks to a fossil reference system considering a specified processing route, and those who focus on comparing different processing routes for biorefinery products [7].

These choices affect the determination of the system boundaries, which in turn determines which processes and flows are included in the life cycle inventory (LCI). Figure 2 shows the difference in system boundaries outlined in several studies of biorefineries, considering different feedstocks and end products. It can be seen that the selected system boundaries varied significantly amongst LCA studies of biorefinery and AD systems; for example, while study F [49] only considered the production of infrastructure and the end-of-life phase, study W [50] only considered the cultivation and harvesting of the biomass feedstock. Such decisions can influence the focus and findings of the study, particularly in the case of environmental hotspot analysis. For example, study P [51] and study V [52] both analyzed the use of miscanthus as a feedstock for bioenergy generation. However, [51] considered the pre-treatment (pelletization) of miscanthus, while [52] considered the conversion (combustion) of miscanthus into bioenergy. While Shen et al. [52] acknowledged the benefits of pelletization on reducing the weight of biomass for transport, they did not consider it within the LCA study. Thus, [52] found that both transport and conversion had a large impact on the life cycle, while [51] determined that pelletization contributed the most to the life cycle impacts and transportation had a relatively minimal impact. From the figure it is also notable that the inclusion of life cycle emissions related to biorefinery infrastructure and capital equipment is less common. This is generally due to the time-intensive process of gathering infrastructure data compared to low expected life cycle impacts [33]. Even when the production of capital goods is included, studies often limit the included capital goods to certain life cycle stages [10], exclude the construction of buildings or roads [49,53], or do not consider the final dismantling of the plant [54].

Figure 2. Comparison of system boundaries and inclusions of several LCA studies utilizing biomass feedstocks. Sources: A [55]; B [13]; C [56]; D [57]; E [11]; F [49]; G [54]; H [58]; I [20]; J [59]; K [60]; L [61]; M [62]; N [63]; O [64]; P [51]; Q [65]; R [14]; S [53]; T [66]; U [10]; V [52]; W [50]; X [67]; Y [8].

*Transport F/P: transportation of feedstocks and/or products to/from biorefinery. Production CE: production of capital equipment/infrastructure. Transport CE: transportation of capital equipment.

One choice of particular importance is the functional unit (FU), which is also determined in the goal and scope definition. The functional unit is the unit to which all impacts are held relative to, and is often based on the product or the service provided by that product [68]. Some standards can have specific requirements for the functional unit, such as 1 MJ fuel for the Renewable Energy Directive (RED) [69]. If the goal is to understand or evaluate the use of resources, the functional unit can also be based on inputs such as feedstock or land use [27]. For biorefineries, functional units can combine several products in order to capture the multi-functionality of the system, or multiple functional units can be used to capture multiple aims for the study [11]. In any case, the functional unit chosen should be consistent with the goal and scope of the study.

3.2 Allocation of biorefinery outputs and biomass feedstocks

Allocation refers to the splitting or partitioning of environmental burdens to a product or system [70]. Different standards make different suggestions on the use of allocation. The ILCD Handbook [45] only suggests allocation in some goal situations whereas the ISO standards [43,44] suggest defining the goal in a way which avoids allocation altogether. In either case, the alternative is system expansion, which is performed through substitution or system enlargement [27].

To give a better understanding of these concepts in the context of biorefineries, an example is given of a biorefinery producing products A, B and C (Figure 3). If the goal is to determine the impacts of producing product A, substitution can be used. All of the impacts of the biorefinery system are allocated to product A, but products B and C are assumed to displace an equal amount of products on the market. Thus, the impacts from other systems producing B and C are subtracted from the impacts of the biorefinery system. If the goal of the LCA is to determine the impacts of the biorefinery system as a whole and compare it to other systems, system enlargement can be used. With this method, impacts are calculated for all products leaving the biorefinery (A, B, and C) and compared to the impacts of one or multiple systems added together which would produce an equal amount of the three products. If the goal of the biorefinery is to understand the impacts of each product separately, allocation is needed. Some percentage of the impacts from the biorefinery system should be allocated to product A, some percentage to product B, and some percentage to product C. How these percentages are determined is the concept of allocation. For example, 20% of the impacts from the biorefinery system could be associated to product A, 30% to product B, and 50% to product C.

Figure 3. Example of a. substitution b. system enlargement and c. allocation of products in a biorefinery system.

In LCA studies of bio-based processes, substitution is often used where avoided impacts from replacing a fossil-based product with a bio-based product are subtracted from the total environmental impacts [10,57]. The inclusion of avoided impacts in LCA modelling has a considerable effect on the total environmental impact calculated [64]. This could be misleading when comparing the results of several studies; for example, an LCA which uses substitution would compare the impacts of final products to fossil-based products, typically resulting in negative values per functional unit [9,24,54,65], whereas an LCA which does not use substitution would simply account for the impacts of the products, typically resulting in positive values per functional unit [46,52,66,67].

If allocation is unavoidable, it can be done based on certain physical or economic values. For example, the Renewable Energy Directive (RED) requires allocation based on the lower heating value (LHV) of the products [69]. For use in environmental footprinting and Chain of Custody models, mass allocation is required. For example, in industrial settings where blending of certified and non-certified products is performed, the amount of final products to be certified must be equal on a mass basis to the amount of certified ingredients used in their production [71]. Due to the multifunctional nature of biorefineries, the method used to handle allocation is a significant decision. For example, in the study by Cai et al. [11], multiple scenarios for the life cycle analysis of a biorefinery were tested, using three categories of allocation methods (system-level allocation, process-level allocation, and displacement) and four categories of allocation basis (mass-based, market value-based with current market value, market value-based with 50% market value, and carbon efficiency). When considering the co-production of succinic acid or adipic acid from a biodiesel-producing biorefinery, the authors found that allocation methods could change the resulting life cycle GHG emissions substantially (Table 2). In another study by Timonen et al. [67], the production of biogas and digestate from AD processes was assessed. The authors showed that allocation by energy, economic value, or mass could shift environmental burdens considerably (Table 3).

Table 2. Overview of results from Cai et al. [11], showing climate change impacts (g CO2-eq) per functional unit (MJ RDB) depending on biorefinery design (columns) and allocation method (rows).

|  | Renewable diesel blendstock (RDB) production only | Succinic acid co-production design | Adipic acid co-production design |
| --- | --- | --- | --- |
| Base case (no allocation necessary) | 61 g CO2-eq/MJ RDB | - | - |
| System-level allocation | - | 43-126 g CO2-eq/MJ RDB | 49-85 g CO2-eq/MJ RDB |
| Process-level allocation | - | 32-72 g CO2-eq/MJ RDB | 30-47 g CO2-eq/MJ RDB |
| Displacement (system expansion) | - | -97 g CO2-eq/MJ RDB | -272 g CO2-eq/MJ RDB |

Table 3. Overview of results from Timonen et al. [59], showing climate change impacts (g CO2-eq) per each product (energy produced with a functional unit of MJ and nitrogen-rich fertilizer produced with a functional unit of g N) depending on feedstock inputs (columns) and allocation method (rows).

Scenario 1: AD of 16000 t/a pig slurry and 3500 t/a solid fraction of separated pig slurry. Scenario 2: AD of 16000 t/a pig slurry and 3500 t/a grass from uncultivated fields. Scenario 3: AD of 16000 t/a pig slurry and 3500 t/a side streams from the food industry.

|  | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| Energy allocation | 18.4 g CO2-eq/MJ 0 g CO2-eq/g N | 19.6 g CO2-eq/MJ 0 g CO2-eq/g N | 14.9 g CO2-eq/MJ 0 g CO2-eq/g N |
| Economic allocation | 6 g CO2-eq/MJ 1.5 g CO2-eq/g N | 7.2 g CO2-eq/MJ 1.6 g CO2-eq/g N | 4.7 g CO2-eq/MJ 1.1 g CO2-eq/g N |
| Mass allocation | <1 g CO2-eq/MJ 2.2 g CO2-eq/g N | <1 g CO2-eq/MJ 2.4 g CO2-eq/g N | <1 g CO2-eq/MJ 1.5 g CO2-eq/g N |

3.3 Land and sea surface use

In LCA, the consideration of land use should be determined at the goal and scope stage but will subsequently affect the life cycle inventory (LCI) and life cycle impact assessment (LCIA) stages. Land can be represented by land use/land occupation or land use change/land transformation [27]. Land use change can be further categorized as direct land use change (dLUC) or indirect land use change (iLUC), where iLUC is associated with consequential modelling [45].

While LCIA methodologies have been developed in recent years which attempt to quantify the impacts of land occupation and transformation, significant consensus on how to address land use impacts has yet to be achieved [72]. Land use change can contribute to climate related emissions due to changes in soil carbon stocks, which can be addressed in LCIA by the framework provided by the International Panel on Climate Change [IPCC] [45]. In terms of effects on biodiversity, LCIA methodologies often attempt to address impacts of land use through endpoint indicators [73]. These methodologies have some significant shortfalls; first, accounting for land occupation and transformation is challenging due to the necessity to understand what other potential uses for that land exist in the region for which the LCA is conducted, and how long after its use it will take to return to its “natural” state [74]. Furthermore, it is argued that the current methods for approximating biodiversity impacts are limited and oversimplified, without a full consideration of complex ecosystem dynamics [34].

In the case of marine-based feedstocks, the cultivation is not dependent on land; however, the utilization of marine resources also has impacts and in fact are currently underrepresented in LCA methodology [35]. An equivalent indicator in this case could be sea surface area occupied, but as consideration of sea surface occupation and transformation have yet to be addressed in standardized LCA methodology, LCA studies which consider sea surface use as an impact category have needed to develop their own unique methodologies, increasing complexity and barriers to adoption [33].

3.4 Biogenic carbon and timing of emissions

A large driver for the bioenergy industry is the replacement of fossil feedstocks, which require a geological timescale to form, with biomass (biogenic) feedstocks, which can be produced in a much shorter time period [27]. This leads most standardized LCAs to account for biogenic carbon emissions separately from fossil carbon emissions, and furthermore to an assumption that bioenergy is carbon-neutral and thus biogenic emissions do not need to be included in LCA [45]. For example, while carbon emitted from products such as fossil fuel-derived fertilizers and diesel would be considered in the LCA, carbon emitted from organic fertilizers and biodiesel would not. However, just as carbon from fossil fuel combustion is emitted to the atmosphere at a different time scale than its storage via fossilization, carbon emissions from the utilization of biomass feedstocks occur at a different time scale than carbon sequestration from its growth [75].

Because standard LCIAs are conducted by aggregating emissions over a product’s lifetime, the exclusion of temporal factors in LCA is a key issue in its accuracy [41]. As one study points out, “Releasing a big amount of pollutant instantaneously generally does not have the same impact as releasing the same amount of pollutant at a small rate over several years” [75 p3169]. Recent studies have shown that the inclusion of temporal factors in LCAs involving biomass can change the assessment results considerably [39,76]. Thus, for the LCA of biorefinery systems, research suggests that timing and impact of greenhouse gas (GHG) emissions from bioeconomy activity should be considered separate to the cultivation of its feedstocks [27].

3.5 Impacts assessed, normalization and weighting

The mandatory steps for the LCIA phase are outlined by the ISO standards [43,44]. First, impact categories, indicators and characterization models should be selected in the goal and scope stage of the study. Later, LCI results should be assigned to impact categories. Finally, results for the impact categories should be calculated.

Various methods for LCIA have been developed by organizations, defining the impacts to be assessed and methodology for calculating them. The chosen indicators should be suitable for the intended application and region of study [77]. Generally, there is a distinction between midpoint indicators, which quantify impacts to categories such as climate change, eutrophication, and particulate matter formation, and endpoint indicators, which quantify impacts to broader areas of protection such as human health, natural environment, and natural resources [45]. Table 4 displays the impact assessment categories included in several of the LCA case studies identified through this review. It is clear that midpoint indicators are more frequently utilized, with only 4 case studies found which displayed endpoint results [14,53,54,78]. This could be because midpoint indicators are considered to be more certain and transparent than endpoint indicators [79]; midpoint indicators typically use a mass or energy equivalent to sum the impacts of the life cycle inputs and outputs, such as kg CO2-eq for global warming potential (GWP) or MJ-equivalent for fossil energy use, while endpoint indicators use more abstract units, such as disability-adjusted life years (DALY) for human health impacts [77]. It can also be observed that several studies focus solely on climate change through quantifying GWP [11,49,52,63,67,80]. This may be appropriate depending on the goal and scope of the study, but it is argued that more impact categories should be assessed to give a representative of a product’s environmental performance [81]. Finally, while most studies used typical midpoint and endpoint indicators, there was one notable exception. To assess and compare the impact of seaweed cultivation systems in two different regions, Taelman et al. [50] used Cumulative Exergy Extraction from the Natural Environment (CEENE), a methodology using exergy-based indicators. Exergy is a thermodynamic unit which measures the work obtainable from a system or resource “as it is brought to equilibrium with a reference environment” [50 p178]. In LCA studies, it has been discussed as an alternative method to quantify impacts from a resource consumption perspective [34]. More details regarding the LCA studies reviewed, including feedstocks, products, regional context, functional unit and resulting quantified impacts, is presented in Tables S1 and S2 (Supplementary material).

Table 4. Impact categories included in bio-based LCA studies identified in this review.

| Source | Type of assessment | Impact categories |
| --- | --- | --- |
| [8] | Midpoint | ADP, Acid, Eutro, GWP, ODP, HT, ET-F, ET-M, ET-T, PO, Energy-Ren, Energy-FF |
| [9] | Midpoint | GWP, Eutro, Energy-FF and Nuc, LO-Agri, ET-F |
| [10] | Midpoint | GWP, Energy-total, Energy-FF, Eutro-M, Eutro-M (P-limited), HT-C, HT-NC |
| [11] | Midpoint | GWP |
| [13] | Midpoint | Energy-FF, GWP |
| [14] | Midpoint and endpoint | Midpoint: GWP, ODP, Rad, PM, Acid-T, Eutro-F, Eutro-M, ET-T, ET-F, ET-M, HT-C, HT-NC, LU, MiU, FDP, WU Endpoint: Human health, Ecosystem quality, Natural resources |
| [15] | Midpoint | GWP, ODP, Acid-T, Eutro-F, Eutro-M, HT, PO, PM, ET-T, ET-F, ET-M, Rad, LU-Agri, LU-Urban, LUC-Nat, WU, MeU, FDP |
| [16] | Midpoint | ADP, Acid, Eutro, GWP, ODP, PO, Energy-FF, Energy-Nuc |
| [17] | Midpoint | GWP, ODP, HT, PO, PM, Rad, Acid-T, Eutro-F, Eutro-M, ET-T, ET-F, ET-M, LU-Agri, LU-Urb, LUC-Nat, WU, MeU, FDP |
| [24] | Midpoint | GWP, Eutro, Energy-FF and Nuc, LO-Agri, ET-F |
| [49] | Midpoint | GWP |
| [50] | Other | Exergy-Ren, Exergy-FF, Exergy-Nuc, Exergy-metal, Exergy-minerals, Exergy-Water, Exergy-Land, Exergy-Atm, Exergy-Marine |
| [51] | Midpoint | Acid, Eutro, GWP, Energy-total |
| [52] | Midpoint | GWP |
| [53] | Endpoint | Human health, Ecosystem quality, Climate change, Resources |
| [54] | Midpoint and endpoint | Midpoint: RI, GWP, ET, Acid/Eutro, LU, MiU Endpoint: Human health, Ecosystem quality, Natural resources |
| [55] | Midpoint | Energy-FF, GWP |
| [56] | Midpoint | GWP, Acid, Eutro, ODP, PO, HT |
| [57] | Midpoint | GWP, Acid, Eutro-T |
| [58] | Midpoint | ADP, GWP, ODP, HT, ET-F, ET-M, ET-T, PO, Acid, Eutro |
| [59] | Midpoint | GWP, Acid, Eutro-M, Eutro-T and Eutro-F |
| [60] | Midpoint | GWP, Acid, Eutro-F, Eutro-M, ADP, ET-T, ET-M, ET-F, HT, ODP, PO, WU, MeU, Rad, PM, LU-Agri, LU-Urb, LUC-Nat |
| [61] | Midpoint | ADP, Energy-FF, Acid, Eutro, GWP, HT, ET-F, ET-M, ET-T, ODP, PO |
| [62] | Midpoint | FF, RI, Acid/Eutro, GWP, LU, MU, HT-C, ET, Rad, RO, ODP |
| [63] | Midpoint | GWP |
| [64] | Midpoint | ADP, GWP, ODP, PO, Acid, Eutro, Energy-total |
| [65] | Midpoint and other | Midpoint: GWP Other: absolute global temperature potential |
| [66] | Midpoint | GWP, FF depletion, HT, ET-F, Eutro-F |
| [67] | Midpoint | GWP |
| [78] | Midpoint, endpoint, and other | Midpoint: LU-Agri, GWP, FF depletion, ET-F, Eutro-F, HT, Rad, ET-M, Eutro-M, MeU, LUC-Nat, ODP, PM, PO, Acid-T, ET-T, LU-Urb, WU Endpoint: Ecosystem quality, Human health, Natural resources Other: temperature change |
| [80] | Midpoint | GWP |
| Budzinski [82] | Midpoint | Eutro-F, PM, ET-F, HT, Rad, CC-HH, CC-ES, LU-Agri, LUC-Nat, ODP, PO, ET-M, ET-T, Acid-T, LU-Urb, FDP, MiU |
| Murphy 2014 [83] | Midpoint | GWP, Energy-total |
| Murphy 2016 [84] | Midpoint | GWP, Acid, Eutro, Energy-total |
| Pergola [85] | Midpoint | GWP, Acid, PO, Eutro, ET, ODP |

According to the ISO standards, LCIA can also include the optional steps of normalization and weighting [77], where normalization is a method for calculating the impact of one category relative to a reference and weighting is a method for converting and combining different indicators across impact categories [44]. While several studies were found which applied weighting [53,54,62,82], the application of weighting in LCIA is a controversial topic, with the ISO 14044 standard discouraging weighting in comparative LCA studies as it is considered to be not scientifically based and heavily influenced on value choices [44]. Nonetheless, it could be argued that LCA studies are full of subjective choices [37], and weighting approaches can add a valuable sociopolitical perspective to the study [45]. For example, LCIA methodologies such as ReCiPe [86] use normalization and weighting methods to convert midpoint indicators into endpoint indicators, and aggregate endpoint indicators into a single score based on a hierarchical, individualist or egalitarian perspective. Giwa [62] showed that applying these different perspectives could significantly affect the relative performance of the biorefinery.

3.6  Simplification of feedstocks and processes

To simplify an LCA, processes and systems are often represented as a black-box, where inputs and outputs are held constant [36,87]. Process simplifications in the LCA of biorefineries can include lumping together multiple sequential processes into one, such as defining a single cultivation phase which includes sowing, tilling, fertilization, harvesting and baling [83]. Furthermore, biomass transportation is often simplified as a single process with a specified distance, despite the fact that supply chains typically involve multiple suppliers and downstream processing plants [84]. In addition, simplifications such as a constant electricity consumption are assumed despite variable processes [87].

Most notably, the same biorefinery processes are assumed to be used for the entire life cycle, despite highly variable feedstock composition and availability [7]. As seen from the case studies reviewed in this work, biorefinery processes such as AD can utilize a large range of feedstocks, from lignocellulosic feedstocks to macroalgae. Lignocellulosic biomass encompasses a wide range of biomass types such as wood [82], grass [20], and agricultural residues [56], which can contain cellulose from 30-50% dry weight, hemicellulose from 20-40% dry weight, and lignin from 5-25% dry weight [88]. On the other hand, macroalgal biomass feedstocks can contain protein from 5-47% dry weight [89]. Differences such as species, plant age, plant part, and growing conditions such as soil type, altitude, temperature, and shading can lead to significant variations in composition [90-95]. Even considering a specific species and growing conditions, compositions can vary widely depending on the time of year harvested; for example, samples of seaweed species Laminaria digitata collected from the same location off the coast of Scotland were found to range from 75-90% moisture content, 8-27% mannitol, and 2-17% laminarin depending on the time of year [96].

An issue with wide variations in composition is that assumptions can lead to inaccurate inventory modelling. For example, if an assumption was made for an ethanol producing biorefinery which assumed a higher cellulose content and a lower moisture content for a biomass feedstock than reality, the resulting LCA would show a much higher output and lower impact. This assumption could even cause the stakeholder to choose a feedstock or processing pathway which could differ from the optimal one. For example, in the research of Corona et al. [20], several green biomass feedstocks were compared based on their farm inputs and corresponding yields, crude protein content, fiber content, and conversion efficiency to the final biorefinery products. However, the feedstocks were assessed based on protein contents within a narrow range (19.5-21.6%); by comparison, the protein content found in the grass species cocksfoot was found to range from 6% [95] to 22.4% [90] in the literature. If data from another harvest had been compared, a different feedstock could have easily been determined to be the optimal choice.

3.7 Regionality

Life cycle assessments are regionally dependent; thus, the scope of the LCA should include a specification on regional context [45]. The location of a biorefinery system affects the available feedstocks and resources, viable products, and supply chains [97]. One key resource is land and water territories, which varies largely from country to country, as can be seen in the case of the EU (Figure 4). For example, the marine Exclusive Economic Zone (EEZ) controlled by Ireland is 7.5 times larger than that controlled by Germany [98], while the total land area of Ireland is 4.2 times smaller than Germany [99]. Furthermore, the proportion of this land and water that can be used to produce biomass can vary widely. Of the currently utilized agricultural area in the EU, the proportion which is considered permanent grassland ranges from 10% in Denmark to 48% in Austria to 90% in Ireland, while the proportion considered as arable land ranges from 89% in Denmark to 49% in Austria to 10% in Ireland [100]. When utilizing biomass residues, regional trends are also apparent; for example, the theoretical potential of utilizing forest residues within Europe is highest in Sweden and Finland, while the potential of utilizing agri-industrial process waste is highest from regions of Spain corresponding to the olive oil industry [25].

The production of resources and market value of products can also be region-specific and affect the environmental impact of a biorefinery. Ögmundarson et al. [14] showed that for biorefineries using electricity-intensive processes, the regional electricity mix impacts the environmental impacts considerably. On another note, while heat generated from AD could also be sold as a useful product in some regions [101], in other regions excess heat may not be utilized due to a lack of nearby added demand [56,61,62]. Finally, transport considerations are extremely variable in LCA studies of biorefinery systems, ranging from the exclusion of any transport-related emissions [13,14,55] to the inclusion of emissions from the transport of capital goods [53,54,62]. In fact, some studies found that transportation was one of the highest contributors to overall environmental impact in bio-based systems [62,83].

Figure 4. Share of EU land [99] and water [98] territories by country*.

*Excluding Greenland and Faroe Islands (Denmark), Canary Islands, Ceuta and Melilla (Spain), and Madeira and Azores Islands (Portugal).

3.8 Future foreground and background systems

Standard LCAs, both attributional and consequential, aggregate inputs and outputs over a considered lifetime [41]. One issue with this which has already been discussed in section 3.1.4 is the differences in the rate of sequestration and emission over time. The second issue is that while LCAs consider the impacts of a product system over a specified time period, they assume that the system does not develop over time. Thus, a limitation for conventional LCA methodologies is that they do not account for a changing world.

System dynamics in a LCA context can be broken into two categories: changes in foreground systems, i.e. changes in the technology or processes within the system boundary, and changes in background systems, i.e. changes which occur outside the system boundary [45]. An example of a foreground system change could be increasing a process’s energy efficiency while a background system change could be the development of an electrical grid with an increased share of renewables [102]. Accounting for future developments in foreground and background systems is particularly important in meso-macro level LCAs and in the LCA of emerging technologies [45].

4	Current challenges in s-LCA, LCC, and integrated sustainability assessments

As indicated in previous sections, LCA methodologies are typically focused on environmental aspects through quantifying various environmental indicators. However, the environment is only one of three pillars of sustainability, with society and economy representing the other two [1]. This section will discuss social and economic aspects of sustainability, and how they can be evaluated using a life cycle approach. The challenges with integrating all three dimensions of sustainability with LCA methodologies will then be highlighted.

4.1 Social sustainability and s-LCA

Social sustainability refers to the social costs and benefits of a product, process or system, and can be assessed using a life cycle approach with s-LCA [28]. While the guidelines set by the UN Environmental Programme [103] have helped to standardize s-LCA methodology, key challenges in s-LCA execution can be seen. First, as in e-LCA, regional context is a key aspect of s-LCA [104-107]. Thus, for each regionally-specific s-LCA, Siebert et al. [107] argue that indicators should be carefully selected through sequential steps of screening, review of local case studies, stakeholder interviews and feasibility assessment. Second, it is noticeable that metrics utilized for assessing social sustainability are typically limited to macroeconomic indicators, such as the number of jobs created or maintained [108,109]. However, indicators for social sustainability are numerous and varied, from child labor to cultural heritage to corruption [103]. For some indicators, the exclusion is due to a lack of availability of social data, which has been identified as a clear issue for s-LCAs [110]. While tools such as the Social Hotspots Database (SHDB) have been created in recent years to address this gap, the available data is generally limited to a region and sector and does not include data on specific plants, production sites or products [104]. For other indicators, metrics can be difficult to quantify, and thus attempts to incorporate them can seem unreliable or impractical [29].

4.2 Economic sustainability and LCC

Economic sustainability refers to the economic performance of a product, process or system, and is typically assessed by profit and payback periods for individual projects [104], or by turnover or value added on the macro-sectoral scale [108]. Economic assessments are used to ensure that the benefits of a project outweigh the costs, that the most profitable project is chosen amongst all alternatives, and that the use of resources is optimized [111]. Several methods and metrics can be used within economic assessment, including return on investment, payback period, net present value, and internal rate of return [111].

Economic analyses which utilize a life cycle approach are generally referred to as Life Cycle Costing (LCC) [112]. For example, LCC is used in levelized cost of energy (LCOE) calculations, which determine the average cost of energy produced by different technologies by dividing the total lifetime costs by the total lifetime energy production [113]. However, a key challenge with the economic assessment of biorefineries is that multiple products are produced. Thus, as discussed in section 3.1.2, the impacts – in this case costs - from a single shared process should be appropriately divided and allocated to the different products. Furthermore, while LCC frameworks are typically applied considering a single stakeholder perspective [42], the life cycles of bio-based products typically involve multiple stakeholders with conflicting economic interests [114]. For example, in a simplified case of biomass supply to a biorefinery, the stakeholders involved could be the producer of biomass (farmer) and the biomass processor (biorefinery operator). While the farmer has a more profitable enterprise at a higher selling price for the produced biomass, the biorefinery operator has a more profitable enterprise at a lower biomass price. For these reasons, LCCs are typically case or industry specific, leading to the development of various divergent LCC frameworks [115]. Furthermore, Kambanou [115] argues that within those frameworks complexity and uncertainty are added from the steps of scoping, compiling of cost inventory, estimating of costs, and interpretation. Additional criticisms of using traditional economic assessments in sustainability assessments revolve around the utilization of an individual business’s profit potential as the sole metric for assessing the economic performance of a system, which neglects both non-monetary costs to the business and monetary and non-monetary costs shifted to employees, other businesses, and society [116,117].

4.3 Multi-dimensional assessments

As discussed in section 3.1.5, the e-LCA framework already incorporates some social aspects through the endpoint indicators covered by conventional LCA. Including monetary values in the e-LCA framework can also be somewhat integrated i.e. through economic allocation. However, these do not capture the complexity of social and economic life cycle impacts, which can be better represented through s-LCA and LCC, respectively. An integrated life cycle assessment which considers all three dimensions of sustainability is referred to as a Life Cycle Sustainability Assessment (LCSA) and is encouraged, as it brings a holistic perspective to sustainability for both the environment and society [112].

LCSAs typically combine e-LCA, s-LCA and LCC to better understand trade-offs between the environmental, economic and social impacts of a product, process or system. However, each methodology is distinct and thus integrating these dimensions comes with inherent challenges [42]. First, both LCC and s-LCA lack established frameworks using a consequential approach, which Fauzi et al. [42] argue makes them inappropriate for integration with consequential e-LCA. Even for attributional approaches, issues arise. While e-LCA and s-LCA share a standardized framework, they are not often included in the same case study. One issue is that the scope of most s-LCA databases is limited to a societal/sectoral level [104], while multiple e-LCA databases are available for specific products, processes or systems. Another issue is that the level of confidence in indicators’ weights is much higher in e-LCA, with the importance of social indicators less clear and social targets open to debate [118]. For LCC and s-LCA, contrasting stakeholder perspectives is a major barrier to integration; for example, a corporate perspective is common in LCC while s-LCA could take the perspective of workers and consumers [42]. Finally, despite the fact that LCC and e-LCA can co-exist within the same study [85,119], they are not often integrated but performed separately. One issue that impacts on integration is discrepancies between temporal considerations; i.e., conventional LCA assumes a steady-state aggregated over a lifetime while LCC considers discounted money values and is thus temporal by default [42].

5	Recent developments in LCA methodologies

The challenges discussed in sections 3.1, 3.2, and 3.5, while important, are largely tied to methodological choices defined in mainstream or streamlined LCA frameworks and have been discussed at length in previous reviews [7,27,36,37]. Simply, the methodological choices made in each LCA step should always be clear and obvious for the audience, which may include other practitioners looking to compare their own studies. This section will instead focus on developments in LCA methodologies which address the challenges discussed in sections 3.3, 3.4, 3.5, 3.6, 3.7 and 3.8, as well as the challenges of including social and economic aspects discussed in sections 4.1, 4.2 and 4.3.

A common theme in the review of challenges to LCA as it pertains to biorefinery systems is an issue with capturing variability. Section 3.3 discusses issues in capturing the complexity of changes in land and sea use, while section 3.6 discusses issues in capturing variability in feedstocks and processes. Section 3.7 discusses variability in different regional contexts, and sections 3.4 and 3.8 discuss issues with capturing changes in time. Section 4.1, 4.2 and 4.3 discuss additional issues with regional specificity, temporal considerations, and in varied perspectives on sustainability. A typical suggestion made to address variability in LCA is to use sensitivity analyses, which show how much certain assumptions can influence the results [36]. However, by including a sensitivity analysis, the practitioner does not improve the accuracy of the study in any way; rather, they improve the transparency of the possible inaccuracy of the study. This is because sensitivity analysis is an uncertainty analysis, and while uncertainty is a product of “inaccurate measurements, lack of data, or model assumptions” in attempting to translate real world data into a theoretical model, variability is a pre-existing component of the real world [39 p1387].

To address the issue of variability, it is instead recommended to improve the accuracy of the LCA by expanding the framework into less conventional methods and integrating with additional modelling tools. From reviewing the challenges, three areas for improvement were identified, corresponding to the three main sources of variability: 1) spatial considerations 2) time-dynamic systems and 3) integration of sustainability dimensions. The following sections will discuss these areas and identify methodologies which address these areas.

5.1 Spatial considerations

As discussed in section 3.1.3, the consideration of land use in LCA and its effects on the LCIA is established by the IPCC and several endpoint LCIA methodologies [73]. In developing a sustainable bioeconomy, it is important to consider the region-specific impacts of land occupation and transformation and for practitioners to clearly state how it has been addressed. For example, in one study [65], several plots of unused agricultural land were considered for the cultivation of grass for biomethane production. The authors found that the contribution of the system to climate change depended on the initial soil quality of the land on which the grass was cultivated, where lands with low initial soil carbon were typically able to sequester the most carbon thus leading to environmental benefits, while biomass production on lands with high initial soil carbon in fact led to soil C losses thus contributing to climate change.

With regards to the assessment of biodiversity, issues are still prominent. It is argued that current LCIA methodologies for measuring biodiversity loss are not spatially or temporally accurate [34], and are therefore only relevant for hotspot analysis [72]. If understanding impacts on biodiversity is a key goal for the study, it is recommended that an evaluation using site-specific data should be performed externally to the LCA [72]. In this case, assessments of natural capital and ecosystem services could be used, where ecosystem services refer to the contribution of ecological systems to human wellbeing, and natural capital refers to the monetized value assigned to the resources within that system [120].

The consideration of sea surface occupation in LCA studies is still extremely rare; thus, it is also important for the LCA community to adopt LCIA methodologies which consider the associated impacts when evaluating marine-based feedstocks. One example is in the work of Taelman et al. [58], who developed and applied an exergetic LCIA method which accounts for sea surface occupation in addition to terrestrial land occupation for the cultivation of macroalgae. More studies are suggested to develop sea surface occupation into other midpoint and endpoint LCIA methodologies.

In section 3.1.7, regional variations in biomass feedstock and their supply were discussed. A large driver for the bioeconomy is to utilize local resources and revitalize rural communities [6]. Thus, supply chains for biorefinery systems should be modelled and optimized with not only economic but social and environmental sustainability in mind. For example, Serrano-Hernandez et al. [90] determined the optimal site for a new bioenergy facility considering economic and environmental criteria. Murphy et al. [72] optimized biomass supply chains to multiple demand locations based on feedstock availability, location and production and processing GHG emissions. In the case of a microalgal biorefinery utilizing CO2, Somers and Quinn [74] found that only pure sources of CO2 which could be transported by pipeline from a distance of less than 40km when uncompressed or 100km when compressed were both environmentally and economically viable.

5.2 Time-dynamic systems

Section 3.1.6 pointed out the issues associated with simplified black-box modelling for biorefineries, particularly in those utilizing feedstocks with varying compositions, which could influence the processes determined to be the best performing. To address this issue of variability, it is recommended to expand the level of detail of the LCA, for example by integrating with biochemical process modelling and simulation. The recommendation to combine LCA with process modelling and simulation is reflected by the work of Corona et al. [9] and by Julio et al. [7] in their review. Furthermore, it is seen that process modelling is often combined with a techno-economic assessment and LCA to assess the economic and environmental impacts of biorefinery systems [63,74]. In particular, several studies [13,14,55,60,66,82] use Aspen Plus process modelling in combination with the LCA framework to understand not only economic impacts but environmental burdens of biorefinery systems. Not only does process simulation software provide the ability to adjust variables and optimize for a certain set of parameters but they can also be used for material flow accounting to better represent capital equipment and energy needs [122].Although integrating process modelling with LCA can improve the accuracy of LCI data and integration with other sustainability dimensions, it does not necessarily reflect real-time dynamics. For example, one could create a process model in which the maximum amount of biorefinery products are produced from a fixed amount of feedstock, and integrate those values into a static LCA under the assumption that the same amount of feedstock enters the biorefinery each year. Thus, while process modelling can address the issue of accuracy, it does not address the temporal issues within LCA as discussed in sections 3.1.4. and 3.1.8. Temporal aspects of life cycle modelling have been shown to have significant impacts on life cycle impact assessments [38,39,76]. However, the inclusion of temporal dynamics in recent studies has shown a wide variation in its application within life cycle modelling. Sohn et al. [40] define three categories for dynamism in LCA: dynamic process inventory, dynamic systems inventory, and dynamic characterization. Dynamic process inventory and systems inventory are both applied in the life cycle inventory (LCI) modelling, while dynamic characterization is applied in the life cycle impact assessment (LCIA). Beloin-Saint-Pierre et al. [38] also confirm this categorical split as dynamics related to 1) system modelling or 2) impact assessment.

5.2.1 System dynamics modelling

Dynamic process inventory accounts for the potential for future development in single-unit processes, while dynamic systems inventory accounts for “the potential for future change in components of system processes” [44 p315]. In other words, changes can occur both within individual processes and within the system that contains those processes, where dynamic LCI can account for both. Future changes can refer to both short-term changes such as daily or monthly variations and long-term changes such as resource, market, or technological developments over a products’ life cycle. The purpose of including short-term variations in LCA is to optimize processes based on real-time data [36]. An example of real-time optimization based on dynamic background processes is the controlled release of water from a hydroelectric dam to match peaks in electricity demand [31]. Such could also apply to biorefineries supplying decentralized bioenergy by matching their operations to local energy demand, thereby improving both environmental and economic efficiency. For the real-time optimization of foreground processes, integration with process modelling is again useful. For example, Filletti et al. [87] suggest a web-based tool which could collect dynamic data such as resource consumption and local emissions from a process simulation and integrate it with web-based LCA software. This could be further applied to biorefineries through the application of novel biosensor technology, which could detect the chemical composition at key points in the life cycle [123].

It is important to understand not only short-term variations within the biorefinery system itself but also the potential long-term changes in the technology and markets. For these reasons, both consequential LCA and prospective LCA are useful. A consequential LCA considers the impacts of the studied life cycle on other systems and is thus recommended for meso-macro level LCAs, which can account for larger systemic changes [45]. An example of this from the biorefinery perspective is modelling large-scale biomass supply and conversion to bioenergy to meet future energy targets [84,124]. Consequential approaches can be used not only for modelling environmental impacts but also to capture trade-offs with economic and social impacts [42]. For example, the consequential changes of switching from a conventional product to a biorefinery product could result in a worse economic performance but better social and environmental sustainability (Figure 5).

A shortfall of consequential LCA is that while it considers supply and demand, it does not consider advancements to the technology applied or created. In fact, the ILCD [45] highlights that time-related representativeness in LCA is mostly linked to technological changes. As addressed in section 3.1.8, this is particularly important for emerging technologies. For example, one study [102] has shown that for renewable energy technologies, accounting for future systemic changes such as improved efficiency, increased lifetime, advancements in production processes and external services originating from conventional energy and transport systems will change the impact considerably. To address this, prospective LCAs, also known as anticipatory or ex-ante LCAs, are typically used to compare an emerging technology to mature technologies, accounting for developments in the foreground and background systems [32]. However, making assumptions on the future scale-up and adoption of a technology comes with increased uncertainty [12]. As discussed earlier, uncertainty in LCA can be addressed through the use of sensitivity analysis. Another example of how the uncertainty from these assumptions can be integrated into a study is through the use of scenarios which can include baseline, conservative, and optimistic assumptions [63,124].

Figure 5. Example of a consequential LCA (including social and economic impacts) of a novel biorefinery product. The displacement of the conventional product from the uptake of the biorefinery product leads to less fossil energy use, less impact on water usage, and better conditions for workers, but higher overall costs.

An issue with creating a dynamic LCI is that time-differentiated datasets are currently rare. Thus far, only one commercial software (DyPLCA) includes a time-differentiated dataset, utilizing a graph-based computational framework [38]. In cases where a time-differentiated dataset is not available but historical data can be found, it is possible to build a framework which accounts for changes to the LCI over time. To keep track of such changes, System Dynamics Modelling (SDM) could be beneficial. System dynamics modelling is a methodology used to study “complex nonlinear behavior within systems, often used for simulating new potential behaviors by adding, removing or changing variables, triggers and delays” [125 p5]. Combining system dynamics thinking with LCA has been found to be a useful tool for the developing circular economy [125,126]; however, in this literature review no study was found which applies systems thinking to the LCA of biorefineries. SDM could, for example, be used to monitor the effects of seasonal variations (short-term changes) or ecosystem degradation (long-term changes) on biomass resources. A framework relatable to SDM which has been applied to bioeconomy-related LCA is Agent-Based Modelling (ABM), where individual entities and their interactions are represented to demonstrate how micro level decisions affect macro level changes [127]. For example, Bichraoui-Draper et al. [127] demonstrated the usefulness of Agent-Based LCA by modelling the socioeconomic factors which affected farmer decision making and then applying it to the realistic uptake of environmentally friendly practices. In this way, agent-based modelling is more applicable to consequential LCA, a fact which has been highlighted by several studies [38,40].

5.2.3 Dynamic impact assessment

Dynamic characterization is related to the LCIA stage, in which the time horizon of impacts are distinctly accounted for rather than aggregated [40]. In LCA, three time horizons can be differentiated: the length of the product’s life cycle, the timing and persistence of emissions (dynamic LCI), and the time horizon of the LCIA characterization (time-dynamic LCIA) [39]. Time horizon selection has also been related to the principle of economic discounting [39], where the discount rate applied represents the value of time [111]. For example, Yuan et al. [41] outlined a framework for temporal discounting in LCIA, taking into account time differentiated life cycle emissions (Figure 6). However, the selection of different discount rates or time horizons is seen as a subjective and ethical decision [38,39]. Indeed, some LCIA methods assume different time horizons in their characterization depending on the ethical perspective; for example, ReCiPe considers a methane global warming potential (GWP) of 34 for the Hierarchical perspective (100 years), 84 for the Individualist perspective (20 years) and 4.8 for the Egalitarian (1000 years) perspective [86].

Nonetheless, time-dependent LCIAs can address the issue raised in section 3.1.4 by independently characterizing the sequestration, storage and release of emissions such as carbon from biogenic sources. For example, Nilsson et al. [65] implemented time-dynamic LCIA in their case study and found that due to differences in the biological and chemical processes leading to emissions, the major process contributor to climate impacts changed over time. Furthermore, Shimako et al. [78] used a dynamic LCI to calculate climate change impacts over a 100 year time horizon, demonstrating that due to the short time period between capture and release of carbon dioxide, energy production from microalgae could not be considered as a form of carbon sequestration.

Figure 6. Framework for applying temporal discounting to product life cycle stages in LCIA, adapted from Yuan et al. [41].

5.3 Integration of different dimensions of sustainability

The integration of multiple dimensions of sustainability is important for achieving a holistic assessment. Particularly, an integrated framework can capture complexities of bio-based systems existing in complex environmental and socioeconomic systems [128] and can simultaneously reduce the amount of work for practitioners when applied [122]. Integration can be done through multiple frameworks, resulting in either comparative rankings or quantitative single score indicators. For producing a quantitative indicator, economic and social values can be incorporated into the LCA framework (extended LCIA), or social and environmental values can be represented in the LCC framework (extended LCC). For producing comparative rankings, Multi-Criteria Decision Analysis (MCDA) can be used.

5.3.1 Extended LCIA

An extended LCIA includes multiple socioeconomic impact categories along with conventional e-LCA impact categories [112]. Socioeconomic indicators which could easily be included in the LCI inventory could be € profit FU-1 or jobs created FU-1. Even more detailed indicators such as the type of job or laborer could further inform social impact pathways [129]. The impact categories are then weighted to produce a single score indicator such as Quality Adjusted Life Years (QALY) [129].

To improve the use of extended LCIAs, there is a need to further develop impact pathways relating midpoint indicators to endpoint categories. For example, Neugebauer et al. [117] assert that economic midpoints could be related to microeconomic categories such as profitability (value-added), labor-productivity and business diversity, while economic endpoints should be linked to macroeconomic categories such as economic growth (GDP) and resilience (vulnerability index). It is also important to include social indicators unrelated to macroeconomic metrics, such as human rights infringements or unequal opportunities [129]. However, as mentioned in section 3.2.1, for most socioeconomic indicators reliable quantitative data is difficult to find, and thus it can be difficult to incorporate them into sustainability assessments. Therefore, it is important for researchers to continue gathering regional and system-specific social data for biorefinery systems.

5.3.2 Extended LCC

An extended LCC follows a conventional LCC but also includes external ‘costs’ which can represent monetized effects of environmental and social impacts outside the economic system [130]. Extended LCC therefore has many ties to the Cost-Benefit Analysis (CBA) methodology, a widely established methodology for assessing trade-offs by weighing the monetized costs and benefits of a project to society [131]. The resulting single score indicator is a monetary value, such as € or $.

One significant issue with both extended LCC and CBA is the methodological and ethical debate on putting a price on social justice or environmental protection [130]. One proposal to address the issue of monetizing social and environmental costs is to allow stakeholders to weigh these costs by identifying a willingness-to-pay. Studies have shown that a tangible willingness-to-pay can be identified by consumers of products or services which are affected by environmental factors such as biodiversity and ecosystem health [120]. In the study done by Winden et al. [132], surveys were used to find respondents willingness-to-pay (WTP) for alternative fuels reducing life cycle environmental damage, human health risk and resource depletion. They found an increase in consumer willingness-to-pay of $0.04-0.06 per gallon for bioethanol compared to gasoline.

5.3.3 MCDA

MCDA is a method for evaluating multiple alternatives based on several criteria, where stakeholders rank the performance of each alternative for each criteria [128]. The criteria are then weighed either subjectively or objectively, depending on the selected methodology. The result is a dimensionless ranking of products, processes or systems based on environmental, social and economic sustainability criteria. For example, D’Adamo et al. [108] performed a MCDA using statistical data to rank European countries on the performance of the bioeconomy based on the socioeconomic indicators of value added, turnover and number of workers.

A key influence in MCDAs and in sustainability integration in general is stakeholder inclusivity. In fact, stakeholder engagement is mentioned as one of the most significant issues in LCSA, as the relative weights of the environmental, social and economic impacts included are very influential on the results [112]. Because different stakeholders have different priorities [114], if stakeholder inclusivity is limited, biases can be observed. For example, in their LCSA, Ekener et al. [133] demonstrated that the relative rankings of biomass-based and fossil-based transportation fuels changed depending on the societal perspective of the decision-maker.

In studies using MCDA, it is therefore recommended to include a large and diverse range of stakeholders. The panel chosen should be based in the region for which the biorefinery system is located [107] and should include stakeholder perspectives from all affected groups, such as companies, workers, consumers, society and local community [118]. In the case that stakeholder participation is limited, it is important to clearly state the potential biases of the stakeholder group.

6 	Discussion

In section 5, several methodologies were discussed which addressed issues in variability as defined by spatial differences, temporal considerations, and sustainability integration. Here, a final discussion is provided to summarize the challenges, recommendations, and tools for assessing the sustainability of biorefinery systems.

- The regional context of a biorefinery has significant impacts on its environmental performance. The impacts of emissions, land and water use can be significantly different depending on the region. Biodiversity impacts are also site-specific. Furthermore, the location of the biorefinery affects the transportation of feedstocks, materials, and products.

- Temporal variability can have significant impacts on the sustainability of the system. Temporal variability in LCA can be categorized into dynamic processes, dynamic systems and dynamic characterization. Dynamic processes and dynamic systems are addressed through dynamic LCI, which can be affected by both short-term variations and long-term variations. Dynamic characterization refers to a dynamic LCIA, where temporally-differentiated emissions are quantified and weighted based on the time horizons defined.

- Integration of environmental, social and economic aspects is essential to holistically assess the sustainability of biorefinery systems. Integration requires the application of weights to all assessed indicators and is heavily influenced by variability in stakeholder perspective.

To address these issues, options are available both within the standardized LCA framework and as external tools. Within the standardized LCA framework, time horizons can be defined to better reflect the system being studied. LCIA methodology can be expanded to improve the quantification of impacts from land and sea surface occupation, such as through exergy analysis, and to differentiate between time-dependent sources, such as the sequestration and emission of biogenic carbon. Second, consequential LCA and prospective LCA frameworks can be used to capture long-term variability such as macroeconomic changes and technological developments for dynamic foreground and background systems. Natural capital assessments can better quantify the impacts of changes to ecological systems on biodiversity. To assess the overall sustainability of a bio-based system, the LCSA framework can be used, where quantitative results can be achieved through extended LCC or extended LCIA. Finally, a MCDA including multiple stakeholder perspectives can be used to integrate data obtained from e-LCA, s-LCA and LCC for comparative rankings.

Most notably, integration of LCA with systems modelling tools can improve the inclusion of spatial, temporal, and socioeconomic considerations. Supply chain modelling can be used to better understand transport influences on biorefinery systems, while biochemical process modelling can improve the accuracy of life cycle inventories and the potential for real-time optimization. Agent-based modelling can incorporate stakeholder decision-making considering socioeconomic influences, while system dynamics modelling can quantify the impacts of complex systemic changes.

7 	Conclusions

This paper reviewed LCA studies of AD and biorefinery systems utilizing second and third generation feedstocks, identifying methodological challenges and notable developments for capturing the complexities of bio-based systems. Life cycle assessments of biorefinery systems are abundant, but methodological variations reduce their comparability while simplifications reduce their accuracy. Methodological divergence begins in the goal and scope stage, where key choices must be made regarding the functional unit, system boundaries, geographical boundaries, method of allocation, impacts assessed and assessment method used. Consideration of spatial and time differentiation in LCA also varies significantly, which affects quantification of land use impacts, carbon accounting, and interactions with background systems. Finally, consideration of social and economic aspects through LCA methodologies is limited by lack of diversity in impacts and stakeholders.

Efforts have been made to improve LCA methodologies in these aspects, so impacts can be better quantified and understood. Multiple methodologies have been discussed in this review which contribute to more robust sustainability assessments for biorefinery systems. These include the use of existing LCA methodologies such as consequential LCA, developing methodologies such as prospective LCA, dynamic LCIA and exergetic LCIA, integrated sustainability assessments such as extended LCIA and extended LCC, external tools such as MCDA and natural capital assessment, and systems modelling tools such as supply chain modelling, process modelling, system dynamics modelling and agent-based modelling. Ultimately, practitioners should consider the benefits of utilizing or further developing these frameworks for improving the future applications of life cycle thinking.

Acknowledgments: This research was supported by the European Union’s Horizon 2020 research and innovation programme AgRefine under the Marie Skłodowska-Curie grant agreement No 860477.

References

[1]	Purvis B, Mao Y, Robinson D. Three pillars of sustainability: in search of conceptual origins. Sustainability Science 2019;14;681-95. https://doi.org/10.1007/s11625-018-0627-5.

[2]	Brundtland G. Report of the World Commission on Environment and Development: Our common future. Oxford: 1987. https://sustainabledevelopment.un.org/content/documents/5987our-common-future.pdf.

[3]	Bugge M, Hansen T, Klitkou A. What is the bioeconomy? A review of the literature. Sustainability 2016;8(7);6062. https://doi.org/10.3390/su8070691.

[4]	Ronzon T, Sanjuán AI. Friends or foes? A compatibility assessment of bioeconomy-related Sustainable Development Goals for European policy coherence. Journal of Cleaner Production 2020;254;119832. https://doi.org/10.1016/j.jclepro.2019.119832.

[5]	van der Ven H, Rothacker C, Cashore B. Do eco-labels prevent deforestation? Lessons from non-state market driven governance in the soy, palm oil, and cocoa sectors. Global Environmental Change 2018;52;141-51. https://doi.org/10.1016/j.gloenvcha.2018.07.002.

[6]	European Commission. Bioeconomy: the European way to use our natural resources. Action plan 2018. Luxembourg: 2018. https://doi.org/10.2777/79401.

[7]	Julio R, Albet J, Vialle C, Vaca-Garcia C, Sablayrolles C. Sustainable design of biorefinery processes: existing practices and new methodology. Biofuels, Bioproducts and Biorefining 2017;11:373–95. https://doi.org/10.1002/bbb.1749.

[8]	van Oirschot R, Thomas JBE, Gröndahl F, Fortuin KPJ, Brandenburg W, Potting J. Explorative environmental life cycle assessment for system design of seaweed cultivation and drying. Algal Research 2017;27:43–54. https://doi.org/10.1016/j.algal.2017.07.025.

[9]	Corona A, Ambye-Jensen M, Vega GC, Hauschild MZ, Birkved M. Techno-environmental assessment of the green biorefinery concept: Combining process simulation and life cycle assessment at an early design stage. Science of the Total Environment 2018;635:100–11. https://doi.org/10.1016/j.scitotenv.2018.03.357.

[10]	Seghetta M, Romeo D, D’Este M, Alvarado-Morales M, Angelidaki I, Bastianoni S, et al. Seaweed as innovative feedstock for energy and feed – Evaluating the impacts through a Life Cycle Assessment. Journal of Cleaner Production 2017;150:1–15. https://doi.org/10.1016/j.jclepro.2017.02.022.

[11]	Cai H, Han J, Wang M, Davis R, Biddy M, Tan E. Life-cycle analysis of integrated biorefineries with co-production of biofuels and bio-based chemicals: co-product handling methods and implications. Biofuels, Bioproducts and Biorefining 2018;12:815–33. https://doi.org/10.1002/bbb.1893.

[12]	Holma A, Koponen K, Antikainen R, Lardon L, Leskinen P, Roux P. Current limits of life cycle assessment framework in evaluating environmental sustainability - Case of two evolving biofuel technologies. Journal of Cleaner Production 2013;54:215–28. https://doi.org/10.1016/j.jclepro.2013.04.032.

[13]	Adom F, Dunn JB, Han J, Sather N. Life-cycle fossil energy consumption and greenhouse gas emissions of bioderived chemicals and their conventional counterparts. Environmental Science and Technology 2014;48:14624–31. https://doi.org/10.1021/es503766e.

[14]	Ögmundarson Ó, Sukumara S, Laurent A, Fantke P. Environmental hotspots of lactic acid production systems. GCB Bioenergy 2020;12;19-38. https://doi.org/10.1111/gcbb.12652.

[15]	Mancini E, Arzoumanidis I, Raggi A. Evaluation of potential environmental impacts related to two organic waste treatment options in Italy. Journal of Cleaner Production 2019;214:927–38. https://doi.org/10.1016/j.jclepro.2018.12.321.

[16]	Lijó L, González-García S, Bacenetti J, Fiala M, Feijoo G, Lema JM, et al. Life cycle assessment of electricity production in italy from anaerobic co-digestion of pig slurry and energy crops. Renewable Energy 2014;68:625–35. https://doi.org/10.1016/j.renene.2014.03.005.

[17]	Vázquez-Rowe I, Golkowska K, Lebuf V, Vaneeckhaute C, Michels E, Meers E, et al. Environmental assessment of digestate treatment technologies using LCA methodology. Waste Management 2015;43;442-59. https://doi.org/10.1016/j.wasman.2015.05.007.

[18]	Platt R, Bauen A, Reumeran P, Geier C, Van Ree R, Vural Gursel I, et al. EU biorefinery outlook to 2030: Studies on support to research and innovation in the area of bio-based products and services. Luxembourg: Publications Office of the European Union, 2021. https://data.europa.eu/doi/10.2777/103465.

[19]	European Biogas Association. Methane emission mitigation strategies: Information sheet for biogas industry. 2020. https://www.europeanbiogas.eu/wp-content/uploads/2020/05/Methane-emission-mitigation-strategies-info-sheet-for-biogas-industry.pdf.

[20]	Baiardi D, Morana C. Climate change awareness: Empirical evidence for the European Union. Center for European Studies: 2021. https://www.euroframe.org/files/user_upload/euroframe/docs/2021/Conference/Session%20A2/EUROF21_Baiardi_Morana.pdf.

[21]	Gaffey J, McMahon H, Marsh E, Vehmas K, Kymäläinen T, Vos J. Understanding consumer perspectives of bio-based products: A comparative case study from Ireland and the Netherlands. Sustainability 2021;13;6062. https://doi.org/10.3390/su13116062.

[22]	Thompson P. The agricultural ethics of biofuels: The Food vs. Fuel debate. Agriculture 2012;2(4);339-58. https://doi.org/10.3390/agriculture2040339.

[23]	Lee RA, Lavoie JM. From first- to third-generation biofuels: Challenges of producing a commodity from a biomass of increasing complexity. Animal Frontiers 2013;3:6–11. https://doi.org/10.2527/af.2013-0010.

[24]	Corona A, Parajuli R, Ambye-Jensen M, Hauschild MZ, Birkved M. Environmental screening of potential biomass for green biorefinery conversion. Journal of Cleaner Production 2018;189:344–57. https://doi.org/10.1016/j.jclepro.2018.03.316.

[25]	Hamelin L, Borzęcka M, Kozak M, Pudełko R. A spatial approach to bioeconomy: Quantifying the residual biomass potential in the EU-27. Renewable and Sustainable Energy Reviews 2019;100;127-142. https://doi.org/10.1016/j.rser.2018.10.017.

[26]	Sönnichsen N. Primary energy consumption in the EU by fuel 2019-2020. Statistica: 2021. https://www.statista.com/statistics/236316/primary-energy-consumption-in-the-eu-by-fuel-in-oil-equivalent/.

[27]	Ahlgren S, Björklund A, Ekman A, Karlsson H, Berlin J, Börjesson P, et al. Review of methodological choices in LCA of biorefinery systems - key issues and recommendations. Biofuels, Bioproducts and Biorefining 2015;9(5);606-19. https://doi.org/10.1002/bbb.1563.

[28]	UNEP/SETAC Life Cycle Initiative. Greening the economy through Life Cycle Thinking: Ten years of the UNEP/SETAC Life Cycle Initiative. United Nations Environmental Programme; 2012. https://www.lifecycleinitiative.org/wp-content/uploads/2013/03/2012_LCI_10_years_28.3.13.pdf.

[29]	Organisation for Economic Co-operation and Development (OECD). Meeting policy challenges for a sustainable bioeconomy. Paris: OECD Publishing; 2018. https://doi.org/10.1787/9789264292345-en.

[30]	Moltesen A, Bjørn A. LCA and sustainability. In: Hauschild MZ, Rosenbaum RK, Irving Olsen S, editors. Life Cycle Assessment, Cham: Springer International Publishing; 2018. https://doi.org/10.1007/978-3-319-56475-3_5.

[31]	Turconi R, Boldrin A, Astrup T. Life cycle assessment (LCA) of electricity generation technologies: Overview, comparability and limitations. Renewable and Sustainable Energy Reviews 2013;28;555-65. https://doi.org/10.1016/j.rser.2013.08.013.

[32]	Thonemann N, Schulte A, Maga D. How to conduct prospective life cycle assessment for emerging technologies? A systematic review and methodological guidance. Sustainability (Switzerland) 2020;12(3);1192. https://doi.org/10.3390/su12031192.

[33]	Henriksson PJG, Guinée JB, Kleijn R, de Snoo GR. Life cycle assessment of aquaculture systems—a review of methodologies. The International Journal of Life Cycle Assessment 2012;17;304-13. https://doi.org/10.1007/s11367-011-0369-4.

[34]	Souza DM, Teixeira RFM, Ostermann OP. Assessing biodiversity loss due to land use with Life Cycle Assessment: Are we there yet? Global Change Biology 2015;21:32–47. https://doi.org/10.1111/gcb.12709.

[35]	Woods JS, Veltman K, Huijbregts MAJ, Verones F, Hertwich EG. Towards a meaningful assessment of marine ecological impacts in life cycle assessment (LCA). Environment International 2016;89–90:48–61. https://doi.org/10.1016/j.envint.2015.12.033.

[36]	Beemsterboer S, Baumann H, Wallbaum H. Ways to get work done: a review and systematisation of simplification practices in the LCA literature. International Journal of Life Cycle Assessment 2020;25:2154–68. https://doi.org/10.1007/s11367-020-01821-w.

[37]	Pizzol M, Laurent A, Sala S, Weidema B, Verones F, Koffler C. Normalisation and weighting in life cycle assessment: quo vadis? International Journal of Life Cycle Assessment 2017;22:853–66. https://doi.org/10.1007/s11367-016-1199-1.

[38]	Beloin-Saint-Pierre D, Albers A, Hélias A, Tiruta-Barna L, Fantke P, Levasseur A, et al. Addressing temporal considerations in life cycle assessment. Science of the Total Environment 2020;743;140700. https://doi.org/10.1016/j.scitotenv.2020.140700.

[39]	Lueddeckens S, Saling P, Guenther E. Temporal issues in life cycle assessment—a systematic review. International Journal of Life Cycle Assessment 2020;25:1385–401. https://doi.org/10.1007/s11367-020-01757-1.

[40]	Sohn J, Kalbar P, Goldstein B, Birkved M. Defining temporally dynamic Life Cycle Assessment: A review. Integrated Environmental Assessment and Management 2020;16:314–23. https://doi.org/10.1002/ieam.4235.

[41]	Yuan C, Wang E, Zhai Q, Yang F. Temporal discounting in life cycle assessment: A critical review and theoretical framework. Environmental Impact Assessment Review 2015;51:23–31. https://doi.org/10.1016/j.eiar.2015.01.001.

[42]	Fauzi RT, Lavoie P, Sorelli L, Heidari MD, Amor B. Exploring the current challenges and opportunities of Life Cycle Sustainability Assessment. Sustainability 2019;11(3);636. https://doi.org/10.3390/su11030636.

[43]	International Organization for Standardization (ISO). Environmental Management – Life Cycle Assessment – Principles and Framework (No. ISO 14040:2006). Brussels: 2006.

[44]	International Organization for Standardization (ISO). Environmental Management - Life Cycle Assessment - Requirements and Guidelines (No. ISO 14044:2006). Brussels: 2006.

[45]	European Commission. International Reference Life Cycle Data System (ILCD) Handbook - General guide for Life Cycle Assessment - Detailed guidance. 1st ed. Luxembourg: Publications Office of the European Union; 2010. https://doi.org/10.2788/38479.

[46]	Bjørn A, Laurent A, Owsianiak M, Olsen SI. Goal definition. In: Hauschild MZ, Rosenbaum RK, Irving Olsen S, editors. Life Cycle Assessment, Cham: Springer International Publishing; 2018. https://doi.org/10.1007/978-3-319-56475-3_7.

[47]	Bjørn A, Owsianiak M, Laurent A, Olsen SI, Corona A, Hauschild MZ. Scope definition. In: Hauschild MZ, Rosenbaum RK, Irving Olsen S, editors. Life Cycle Assessment, Cham: Springer International Publishing; 2018. https://doi.org/10.1007/978-3-319-56475-3_8.

[48]	European Commission. The European Commission’s Knowledge Centre for the Bioeconomy: Brief on the use of Life Cycle Assessment (LCA) to evaluate environmental impacts of the bioeconomy. 2019. https://doi.org/10.2760/90611.

[49]	Canter CE, Davis R, Urgun-Demirtas M, Frank ED. Infrastructure associated emissions for renewable diesel production from microalgae. Algal Research 2014;5;195-203. https://doi.org/10.1016/j.algal.2014.01.001.

[50]	Taelman SE, Champenois J, Edwards MD, de Meester S, Dewulf J. Comparative environmental life cycle assessment of two seaweed cultivation systems in North West Europe with a focus on quantifying sea surface occupation. Algal Research 2015;11:173–83. https://doi.org/10.1016/j.algal.2015.06.018.

[51]	Murphy F, Devlin G, McDonnell K. Miscanthus production and processing in Ireland: An analysis of energy requirements and environmental impacts. Renewable and Sustainable Energy Reviews 2013;23:412–20. https://doi.org/10.1016/j.rser.2013.01.058.

[52]	Shen X, Kommalapati RR, Huque Z. The comparative life cycle assessment of power generation from lignocellulosic biomass. Sustainability 2015;7(10):12974–87. https://doi.org/10.3390/su71012974.

[53]	Pilicka I, Blumberga D, Romagnoli F. Life Cycle Assessment of biogas production from marine macroalgae: A Latvian scenario. Environmental and Climate Technologies 2011;6:69–78. https://doi.org/10.2478/v10145-011-0010-6.

[54]	Cappelli A, Gigli E, Romagnoli F, Simoni S, Blumberga D, Palerno M, et al. Co-digestion of macroalgae for biogas production: An LCA-based environmental evaluation. Energy Procedia, vol. 72, Elsevier Ltd; 2015, p. 3–10. https://doi.org/10.1016/j.egypro.2015.06.002.

[55]	Adom FK, Dunn JB. Life cycle analysis of corn-stover-derived polymer-grade l-lactic acid and ethyl lactate: greenhouse gas emissions and fossil energy consumption. Biofuels, Bioproducts and Biorefining 2017;11:258–68. https://doi.org/10.1002/bbb.1734.

[56]	Aitken D, Bulboa C, Godoy-Faundez A, Turrion-Gomez JL, Antizar-Ladislao B. Life cycle assessment of macroalgae cultivation and processing for biofuel production. Journal of Cleaner Production 2014;75:45–56. https://doi.org/10.1016/j.jclepro.2014.03.080.

[57]	Alvarado-Morales M, Boldrin A, Karakashev DB, Holdt SL, Angelidaki I, Astrup T. Life cycle assessment of biofuel production from brown seaweed in Nordic conditions. Bioresource Technology 2013;129:92–9. https://doi.org/10.1016/j.biortech.2012.11.029.

[58]	Cherubini F, Jungmeier G. LCA of a biorefinery concept producing bioethanol, bioenergy, and chemicals from switchgrass. The International Journal of Life Cycle Assessment 2010;15;53-66. https://doi.org/10.1007/s11367-009-0124-2.

[59]	Czyrnek-Delêtre MM, Rocca S, Agostini A, Giuntoli J, Murphy JD. Life cycle assessment of seaweed biomethane, generated from seaweed sourced from integrated multi-trophic aquaculture in temperate oceanic climates. Applied Energy 2017;196;34-50. https://doi.org/10.1016/j.apenergy.2017.03.129.

[60]	Daful AG, Haigh K, Vaskan P, Görgens JF. Environmental impact assessment of lignocellulosic lactic acid production: Integrated with existing sugar mills. Food and Bioproducts Processing 2016;99;58-70. https://doi.org/10.1016/j.fbp.2016.04.005.

[61]	Fusi A, Bacenetti J, Fiala M, Azapagic A. Life cycle environmental impacts of electricity from biogas produced by anaerobic digestion. Frontiers in Bioengineering and Biotechnology 2016;4;26. https://doi.org/10.3389/fbioe.2016.00026.

[62]	Giwa A. Comparative cradle-to-grave life cycle assessment of biogas production from marine algae and cattle manure biorefineries. Bioresource Technology 2017;244:1470–9. https://doi.org/10.1016/j.biortech.2017.05.143.

[63]	Greene JM, Gulden J, Wood G, Huesemann M, Quinn JC. Techno-economic analysis and global warming potential of a novel offshore macroalgae biorefinery. Algal Research 2020;51;102032. https://doi.org/10.1016/j.algal.2020.102032.

[64]	Martínez-Blanco J, Muñoz P, Antón A, Rieradevall J. Life cycle assessment of the use of compost from municipal organic waste for fertilization of tomato crops. Resources, Conservation and Recycling 2009;53(6);340-51. https://doi.org/10.1016/j.resconrec.2009.02.003.

[65]	Nilsson J, Sundberg C, Tidåker P, Hansson P-A. Regional variation in climate impact of grass-based biogas production: A Swedish case study. Journal of Cleaner Production 2020;275;122778. https://doi.org/10.1016/j.jclepro.2020.122778.

[66]	Pachón ER, Vaskan P, Gorgens JF, Gnansounou E. Process design, techno-economic, and life-cycle assessments of selected sugarcane-based biorefineries: A case study in the South African context. In: Refining Biomass Residues for Sustainable Energy and Bioproducts: Technology, Advances, Life Cycle Assessment, and Economics, Elsevier; 2019, p. 567–97. https://doi.org/10.1016/B978-0-12-818996-2.00026-0.

[67]	Timonen K, Sinkko T, Luostarinen S, Tampio E, Joensuu K. LCA of anaerobic digestion: Emission allocation for energy and digestate. Journal of Cleaner Production 2019;235:1567–79. https://doi.org/10.1016/j.jclepro.2019.06.085.

[68]	Ernst & Young and Quantis. Product Carbon Footprinting - a study on methodologies and initiatives. Final report. European Commission DG Environment, 2010. https://ec.europa.eu/environment/eussd/smgp/pdf/Product_Carbon_Footprint_study.pdf.

[69]	European Parliament, the Council of the European Union. Directive (EU) 2018/2001 on the promotion of the use of energy from renewable sources (recast). 2018. http://data.europa.eu/eli/dir/2018/2001/oj.

[70]	Cherubini F, Strømman AH, Ulgiati S. Influence of allocation methods on the environmental performance of biorefinery products—A case study. Resources, Conservation and Recycling 2011;55(11);1070-7. https://doi.org/10.1016/j.resconrec.2011.06.001.

[71]	ISEAL Alliance. Chain of custody models and definitions. A reference document for sustainability standards systems, and to complement ISEAL’s Sustainability Claims Good Practice Guide. Version 1.0. 2016. https://www.isealalliance.org/sites/default/files/resource/2017-11/ISEAL_Chain_of_Custody_Models_Guidance_September_2016.pdf.

[72]	Teixeira RFM, de Souza DM, Curran MP, Antón A, Michelsen O, Milá I Canals L. Towards consensus on land use impacts on biodiversity in LCA: UNEP/SETAC Life Cycle Initiative preliminary recommendations based on expert contributions. Journal of Cleaner Production 2016;112:4283–7. https://doi.org/10.1016/j.jclepro.2015.07.118.

[73]	Vidal-Legaz Beatriz, Antón A, Maia De Souza Danielle, Sala Serenella, Nocita Marco, Putman Ben, et al. Land-use related environmental indicators for life cycle assessment : analysis of key aspects in land use modelling. Publications Office of the European Union; 2016. https://op.europa.eu/en/publication-detail/-/publication/3ade9309-48be-11e6-9c64-01aa75ed71a1.

[74]	Mattila T, Helin T, Antikainen R, Soimakallio S, Pingoud K, Wessman H. Land use in life cycle assessment. Helsinki: 2011. http://hdl.handle.net/10138/37049.

[75]	Levasseur A, Lesage P, Margni M, Samson R. Biogenic Carbon and Temporary Storage Addressed with Dynamic Life Cycle Assessment. Journal of Industrial Ecology 2013;17:117–28. https://doi.org/10.1111/j.1530-9290.2012.00503.x.

[76]	Levasseur A, Lesage P, Margni M, Deschěnes L, Samson R. Considering time in LCA: Dynamic LCA and its application to global warming impact assessments. Environmental Science and Technology 2010;44:3169–74. https://doi.org/10.1021/es9030003.

[77]	Rosenbaum RK, Hauschild MZ, Boulay A-M, Fantke P, Laurent A, Núñez M, et al. Life Cycle Impact Assessment. In: Hauschild MZ, Rosenbaum RK, Irving Olsen S, editors. Life Cycle Assessment, Cham: Springer International Publishing; 2018. https://doi.org/10.1007/978-3-319-56475-3_10.

[78]	Shimako AH, Tiruta-Barna L, Pigné Y, Benetto E, Navarrete Gutiérrez T, Guiraud P, et al. Environmental assessment of bioenergy production from microalgae based systems. Journal of Cleaner Production 2016;139:51–60. https://doi.org/10.1016/j.jclepro.2016.08.003.

[79]	Bare JC, Hofstetter P, Pennington DW, Udo de Haes HA.. Midpoints versus endpoints: The sacrifices and benefits. The International Journal of Life Cycle Assessment 2000;5(6);319-26. https://doi.org/10.1007/BF02978665.

[80]	Somers MD, Quinn JC. Sustainability of carbon delivery to an algal biorefinery: A techno-economic and life-cycle assessment. Journal of CO2 Utilization 2019;30;193-204. https://doi.org/10.1016/j.jcou.2019.01.007.

[81]	Bjørn A, Owsianiak M, Molin C, Laurent A. Main Characteristics of LCA. In: Hauschild MZ, Rosenbaum RK, Irving Olsen S, editors. Life Cycle Assessment, Cham: Springer International Publishing; 2018. https://doi.org/10.1007/978-3-319-56475-3_2.

[82]	Budzinski M, Nitzsche R. Comparative economic and environmental assessment of four beech wood based biorefinery concepts. Bioresource Technology 2016;216;613-21. https://doi.org/10.1016/j.biortech.2016.05.111.

[83]	Murphy F, Devlin G, McDonnell K. Forest biomass supply chains in Ireland: A life cycle assessment of GHG emissions and primary energy balances. Applied Energy 2014;116:1–8. https://doi.org/10.1016/j.apenergy.2013.11.041.

[84]	Murphy F, Sosa A, McDonnell K, Devlin G. Life cycle assessment of biomass-to-energy systems in Ireland modelled with biomass supply chain optimisation based on greenhouse gas emission reduction. Energy 2016;109:1040–55. https://doi.org/10.1016/j.energy.2016.04.125.

[85]	Pergola M, Piccolo A, Palese AM, Ingrao C, di Meo V, Celano G. A combined assessment of the energy, economic and environmental issues associated with on-farm manure composting processes: Two case studies in South of Italy. Journal of Cleaner Production 2018;172;3969-81. https://doi.org/10.1016/j.jclepro.2017.04.111.

[86]	Huijbregts MAJ, Steinmann ZJN, Elshout PMF, Stam G, Verones F, Vieira MDM, et al. ReCiPe 2016: A harmonized life cycle impact assessment method at midpoint and endpoint level. Report I: Characterization. 2016. https://www.rivm.nl/bibliotheek/rapporten/2016-0104.pdf.

[87]	Filleti RAP, Silva DAL, Silva EJ, Ometto AR. Dynamic system for life cycle inventory and impact assessment of manufacturing processes. Procedia CIRP, vol. 15, Elsevier B.V.; 2014, p. 531–6. https://doi.org/10.1016/j.procir.2014.06.024.

[88]	van Erven G, de Visser R, Merkx DWH, Strolenberg W, de Gijsel P, Gruppen H, et al. Quantification of Lignin and Its Structural Features in Plant Biomass Using 13C Lignin as Internal Standard for Pyrolysis-GC-SIM-MS. Analytical Chemistry 2017;89:10907–16. https://doi.org/10.1021/acs.analchem.7b02632.

[89]	Mišurcová L. Chemical Composition of Seaweeds. Handbook of Marine Macroalgae, Chichester, UK: John Wiley & Sons, Ltd; 2011. https://doi.org/10.1002/9781119977087.ch7.

[90]	Abraham EM, Kyriazopoulos AP, Parissi ZM, Kostopoulou P, Karatassiou M, Anjalanidou K, et al. Growth, dry matter production, phenotypic plasticity, and nutritive value of three natural populations of Dactylis glomerata L. under various shading treatments. Agroforestry Systems 2014;88;287-99. https://doi.org/10.1007/s10457-014-9682-9.

[91]	Acar Z, Ayan I, Asci O, Basaran U, Mut H. Biodiversity in morphological properties and nutritional values of forage grass species. Journal of Environmental Biology 2009;30(4);583-9. PMID: 20120500.

[92]	Crush JR, Evans JPM, Cosgrove GP. Chemical composition of ryegrass ( Lolium perenne L.) and prairie grass ( Bromus willdenowii Kunth) pastures. New Zealand Journal of Agricultural Research 1989;32;461-8. https://doi.org/10.1080/00288233.1989.10417918.

[93]	Gismatulina YA, Budaeva V. Chemical composition of five Miscanthus sinensis harvests and nitric-acid cellulose therefrom. Industrial Crops and Products 2017;109;227-32. https://doi.org/10.1016/j.indcrop.2017.08.026.

[94]	Tenikecier HS, Ates E. Chemical composition of six grass species (Poaceae sp.) from protected forest range in Northern Bulgaria . Asian Journal of Applied Sciences 2018;11:71–5. https://doi.org/10.3923/ajaps.2018.71.75.

[95]	Waite R. The chemical composition of grasses in relation to agronomical practice. Proceedings of the Nutrition Society 1965;24:38–46. https://doi.org/10.1079/pns19650008.

[96]	Schiener P, Black KD, Stanley MS, Green DH. The seasonal variation in the chemical composition of the kelp species Laminaria digitata, Laminaria hyperborea, Saccharina latissima and Alaria esculenta. Journal of Applied Phycology 2015;27:363–73. https://doi.org/10.1007/s10811-014-0327-1.

[97]	Serrano-Hernandez A, Faulin J, Pintor JM, Belloso J. Determining an Optimal Area to Locate a Biorefinery under Economic and Environmental Criteria. Transportation Research Procedia, vol. 22, Elsevier B.V.; 2017, p. 95–104. https://doi.org/10.1016/j.trpro.2017.03.014.

[98]	EMODnet. Map of the week - Exclusive Economic Zones 2019. https://emodnet.ec.europa.eu/en/map-week-%E2%80%93-exclusive-economic-zones.

[99]	Eurostat. Share of total area by type and land cover (%), 2018. European Environmental Agency 2021. https://ec.europa.eu/eurostat/statistics-explained/index.php?title=File:Share_of_total_area_by_type_and_land_cover_(%25),_2018_May_2021.png.

[100]	Eurostat. Distribution of Utilised Agricultural Area (UAA) by category. European Environment Information and Observation Network 2018. https://www.eea.europa.eu/data-and-maps/daviz/distribution-of-utilised-agricultural-area.

[101]	Dave A, Huang Y, Rezvani S, McIlveen-Wright D, Novaes M, Hewitt N. Techno-economic assessment of biofuel development by anaerobic digestion of European marine cold-water seaweeds. Bioresource Technology 2013;135:120–7. https://doi.org/10.1016/j.biortech.2013.01.005.

[102]	Pehnt M. Dynamic life cycle assessment (LCA) of renewable energy technologies. Renewable Energy 2006;31:55–71. https://doi.org/10.1016/j.renene.2005.03.002.

[103]	UNEP/SETAC Life Cycle Initiative. Guidelines for Social Life Cycle Assessment of Products. United Nations Environment Programme; 2009. https://www.unep.org/resources/report/guidelines-social-life-cycle-assessment-products.

[104]	Gnansounou E, Alves CM. Social Assessment of Biofuels. In: Biofuels: Alternative Feedstocks and Conversion Processes for the Production of Liquid and Gaseous Biofuels, Elsevier; 2019. https://doi.org/10.1016/B978-0-12-816856-1.00005-1.

[105]	Mattioda RA, Mazzi A, Canciglieri O, Scipioni A. Determining the principal references of the social life cycle assessment of products. The International Journal of Life Cycle Assessment 2015;20;1155-65. https://doi.org/10.1007/s11367-015-0873-z.

[106]	Giraldo JAP, Ortiz Sanchez M, Solarte-Toro JC, Cardona Alzate CA. Economic and social aspects of biorefineries. In: Recent Advances in Bioconversion of Lignocellulose to Biofuels and Value-Added Chemicals within the Biorefinery Concept, Elsevier; 2020, p. 199–231. https://doi.org/10.1016/b978-0-12-818223-9.00008-4.

[107]	Siebert A, Bezama A, O’Keeffe S, Thrän D. Social life cycle assessment indices and indicators to monitor the social implications of wood-based products. Journal of Cleaner Production 2018;172;4074-84. https://doi.org/10.1016/j.jclepro.2017.02.146.

[108]	D’Adamo I, Falcone PM, Morone P. A New Socio-economic Indicator to Measure the Performance of Bioeconomy Sectors in Europe. Ecological Economics 2020;176;106724. https://doi.org/10.1016/j.ecolecon.2020.106724.

[109]	Lehtonen O, Okkonen L. Socio-economic impacts of a local bioenergy-based development strategy – The case of Pielinen Karelia, Finland. Renewable Energy 2016;85;610-9. https://doi.org/10.1016/j.renene.2015.07.006.

[110]	Fontes J. Filling the gaps with the Social Hotspots Database. PRé Sustainability 2015. https://pre-sustainability.com/articles/filling-the-gaps-with-the-social-hotspots-database/.

[111]	Coker K. Engineering economics. In: Fortran Programs for Chemical Process Design, Analysis, and Simulation, Elsevier; 1995. https://doi.org/10.1016/B978-0-88415-280-4.X5000-6.

[112]	UNEP/SETAC Life Cycle Initiative. Towards a Life Cycle Sustainability Assessment: Making informed choices on products. UNEP/SETAC Life Cycle Initiative; 2011. https://www.lifecycleinitiative.org/wp-content/uploads/2012/12/2011%20-%20Towards%20LCSA.pdf.

[113]	U.S. Energy Information Administration [EIA]. Levelized costs of new generation resources in the Annual Energy Outlook 2021. 2021. https://www.eia.gov/outlooks/aeo/pdf/electricity_generation.pdf.

[114]	Berg S, Cloutier LM, Bröring S. Collective stakeholder representations and perceptions of drivers of novel biomass-based value chains. Journal of Cleaner Production 2018;200:231–41. https://doi.org/10.1016/j.jclepro.2018.07.304.

[115]	Kambanou ML. Life cycle costing: Understanding how it is practised and its relationship to life cycle management-A case study. Sustainability (Switzerland) 2020;12:3252. https://doi.org/10.3390/SU12083252.

[116]	Jørgensen A, Hermann IT, Mortensen JB. Is LCC relevant in a sustainability assessment? The International Journal of Life Cycle Assessment 2010;15;531-2. https://doi.org/10.1007/s11367-010-0185-2.

[117]	Neugebauer S, Forin S, Finkbeiner M. From life cycle costing to economic life cycle assessment-introducing an economic impact pathway. Sustainability (Switzerland) 2016;8:1–23. https://doi.org/10.3390/su8050428.

[118]	Martínez-Blanco J, Lehmann A, Muñoz P, Antón A, Traverso M, Rieradevall J, et al. Application challenges for the social Life Cycle Assessment of fertilizers within life cycle sustainability assessment. Journal of Cleaner Production 2014;69:34–48. https://doi.org/10.1016/j.jclepro.2014.01.044.

[119]	Innocenzi V, Cantarini F, Zueva S, Amato A, Morico B, Beolchini F, et al. Environmental and economic assessment of gasification wastewater treatment by life cycle assessment and life cycle costing approach. Resources, Conservation and Recycling 2021;168;105252. https://doi.org/10.1016/j.resconrec.2020.105252.

[120]	Balmford A, Rodrigues ASL, Walpole M, ten Brink P, Kettunen M, Braat L, et al. The Economics of Biodiversity and Ecosystems: Scoping the Science. Cambridge, UK: 2008. https://edepot.wur.nl/8959.

[121]	PRé Sustainability. SimaPro database manual - Methods library. Version 4.15. 2020. https://simapro.com/wp-content/uploads/2020/10/DatabaseManualMethods.pdf.

[122]	Bierer A, Götze U, Meynerts L, Sygulla R. Integrating life cycle costing and life cycle assessment using extended material flow cost accounting. Journal of Cleaner Production 2015;108:1289–301. https://doi.org/10.1016/j.jclepro.2014.08.036.

[123]	Alvarez-Gonzalez G, Dixon N. Genetically encoded biosensors for lignocellulose valorization. Biotechnology for Biofuels 2019;12;246. https://doi.org/10.1186/s13068-019-1585-6.

[124]	Tonini D, Astrup T. LCA of biomass-based energy systems: A case study for Denmark. Applied Energy 2012;99;234-46. https://doi.org/10.1016/j.apenergy.2012.03.006.

[125]	Pinto JTM, Sverdrup HU, Diemer A. Integrating life cycle analysis into system dynamics: the case of steel in Europe. Environmental Systems Research 2019;8;8-15. https://doi.org/10.1186/s40068-019-0144-2.

[126]	Yao L, Liu T, Chen X, Mahdi M, Ni J. An integrated method of life-cycle assessment and system dynamics for waste mobile phone management and recycling in China. Journal of Cleaner Production 2018;187;852-62. https://doi.org/10.1016/j.jclepro.2018.03.195.

[127]	Bichraoui-Draper N, Xu M, Miller SA, Guillaume B. Agent-based life cycle assessment for switchgrass-based bioenergy systems. Resources, Conservation and Recycling 2015;103:171–8. https://doi.org/10.1016/j.resconrec.2015.08.003.

[128]	Halog A, Manik Y. Advancing integrated systems modelling framework for Life Cycle Sustainability Assessment. Sustainability 2011;3(2);469-99. https://doi.org/10.3390/su3020469.

[129]	Weidema BP. The integration of economic and social aspects in life cycle impact assessment. International Journal of Life Cycle Assessment 2006;11:89–96. https://doi.org/10.1065/lca2006.04.016.

[130]	Rebitzer G, Hunkeler D. Life cycle costing in LCM: ambitions, opportunities, and limitations. The International Journal of Life Cycle Assessment 2003;8;253-6. https://doi.org/10.1007/BF02978913.

[131]	European Commission. Guide to Cost-Benefit Analysis of investment projects: Economic appraisal tool for Cohesion Policy 2014-2020. Luxembourg: 2015. https://doi.org/10.2776/97516.

[132]	Winden M, Cruze N, Haab T, Bakshi B. Integrating life-cycle assessment and choice analysis for alternative fuel valuation. Ecological Economics 2014;102:83–93. https://doi.org/10.1016/j.ecolecon.2014.03.008.

[133]	Ekener E, Hansson J, Larsson A, Peck P. Developing Life Cycle Sustainability Assessment methodology by applying values-based sustainability weighting - Tested on biomass based and fossil transportation fuels. Journal of Cleaner Production 2018;181:337–51. https://doi.org/10.1016/j.jclepro.2018.01.211.

Impact category abbreviations: GWP = global warming potential/climate change; Acid = acidification; Eutro = eutrophication (T = terrestrial, F = freshwater, M = marine); ODP = ozone depletion/ozone layer; ADP = abiotic depletion potential; PO = photochemical oxidation/photochemical ozone formation/smog; ET = ecotoxicity (T = terrestrial, F = freshwater, M = marine); Rad = Ionizing radiation; HT = human toxicity (C = carcinogens, NC = non-carcinogens); RI = Respiratory inorganics; RO = Respiratory organics; PM = Particulate matter; LU = land use (Agri = agricultural, Urb = Urban, Nat = Natural); LUC = land use change (Agri = agricultural, Urb = Urban, Nat = Natural); Energy = energy use (FF = fossil energy, Nuc = Nuclear energy, Ren = renewable energy); FDP = fossil fuel depletion; MiU = mineral use; MeU = metal use; WU = water use; Exergy = inherent exergy (Ren = renewable energy, FF = fossil fuel energy, Nuc = nuclear energy, Me = metal, Mi = minerals, W = water, L = land, Atm = atmospheric, M = marine).